Ionic liquid-based dispersive liquid–liquid microextraction

for the simultaneous determination of carbamazepine and lamotrigine  in biological samples

Salumeh Ranjbar, Ameneh Porgham Daryasari* and Mojtaba Soleimani

Department of Chemistry, Lahijan Branch, Islamic Azad University, Lahijan, Iran

P.O.Box 4416939515

- Corresponding author. Tel.: +981342230561; Fax: +981342224756

- E-mail address: porgham54@gmail.com

Abstract

This paper described a new approach for the determination of carbamazepine and lamotrigine  in biological samples by ionic liquid dispersive liquid-phase microextraction prior to high-performance liquid chromatography with ultraviolet detection. The effect of different ionic liquids (ILs) on the extraction efficiency of carbamazepine and lamotrigine were investigated. The highest extraction efficiencies of carbamazepine and lamotrigine were obtained using 30 µL of 1-methyl-3-octylimidazolium-hexafluorophosphate [C8MIM][PF6]. Several factors affecting the microextraction efficiency, such as the type and volume of extracting solvent, type and volume of disperser solvent, salt concentration and pH of the sample solution have been optimized. The calibration plots were linear in the range of 0.1-20 mg L-1 for carbamazepine and 0.3 -40 mg L-1 for lamotrigine with detection limits of 0.04 mg L-1 for carbamazepine and 0.07 mg L-1 for lamotrigine in plasma sample.The results confirm the suitability of the presented method as a sensitive method for the analysis of the targeted analytes in urine and plasma samples.

Keyword: Ionic liquids, Dispersive liquid–liquid microextraction, Carbamazepine, Lamotrigine, Human urine, Human plasma

1. Introduction

One of the most common serious neurological disorders is epilepsy.1 Anti-epileptic drugs (AEDs) are the main form of treatment for epilepsy. Carbamazepine, and newer AEDs like lamotrigine (LTG) are among the first-line medicines for treatment of seizures.2 Lamotrigine (LTG), chemically known as [6-(2,3-dichlorophenyl)-1,2,4-triazine-3,5-diamine], used as monotherapy and as an adjunct with other antiepileptics for treatment of partial and generalized toxic-clonic seizures. Its use as a tranquilizer and in the treatment neurological lesions.3,4

Carbamazepine (CBZ) (5-H-dibenze [b,f] azepine-5-carboxamide), is a first line antiepileptic drug used in the treatment of partial and generalized tonic-clonic seizures.5 The chemical structures of carbamazepine and lamotrigine  are shown in Figure 1.

Figure 1. The structure of carbamazepine and lamotrigine

Most biological samples have complex matrixes and the analytes are typically present at low concentration levels, which are not detectable by the analytical instrument. Therefore, a sample preparation step is generally required to extract, isolate, and concentrate the analytes of interest.

Different analytical techniques that have been used for the determination of lamotrigine include planar chromatography,6 HPLC,7 TLC and HPLC,8 GC,9 HPLC and GC,10 capillary electrophoresis11 and immunoassay.12 High performance liquid chromatography (HPLC-UV, HPLC-DAD) and immunoassay were used for determination of carbamazepine in biological materials. Also, gas chromatography with mass spectrometry and liquid chromatography with mass spectrometry have been reported.13-15

Liquid-liquid extraction (LLE),16 solid-phase extraction (SPE)17 and stir bar-sorptive extraction (SBSE)18 have been developed for the determination CBZ in biological fluids. These methods are time consuming and require substantial amounts of toxic organic solvents. The sample preparation methods employed for lamotrigine involved SPME,19 SPE ,20 protein precipitation (PP),21 LLE22 and microextraction by packed sorbent (MEPS).23

A novel microextraction method called dispersive liquid–liquid microextraction (DLLME) was introduced in 2006.24 DLLME utilizes an extraction solvent and a dispersive solvent to produce a cloudy solution. DLLME has become a very popular technique for the extraction of different compounds.25-27 Generally, the extraction solvent used in DLLME is highly toxic and not environmentally friendly.

In environmental friendly sample preparation methods, it is important to use liquid solvents in reduced amounts, replaced with green solvents or even completely eliminated from the analytical procedure.28 Ionic liquids (ILs) are considered to be “environmental friendly solvents”.29 The immiscibility of ILs in water and their capability to solubilize organic species has made them suitable to extract of compounds.30 Recently, ionic liquid DLLME is very popular.31

In this paper, for the first time, DLLME method using IL as extraction solvent combined with high-performance liquid chromatography has been developedfor the simultaneous determination of carbamazepine and lamotrigine in biological samples. The parameters affecting the extraction efficiency such as the type and volume of extracting solvent, type and volume of disperser solvent, salt concentration and pH of the sample solution have been optimized. The proposed method was successfully applied to determine carbamazepine and lamotrigine  in biological samples.

2. Experimental

2.1. Chemicals and reagents

Carbamazepine and lamotrigine  were obtained from Sobhan Darou Company (Rasht, Iran). Acetone, acetonitrile, methanol and sodium chloride were obtained from Merck Company (Germany). Ionic liquids (ILs) [C8MIM][PF6] and [C6MIM][PF6] were obtained from SIGMA-ALDRICH. IL, [C4MIM][PF6] was obtained from Fluka. Buffer solution (di-sodium hydrogen phosphate-potassium di-hydrogen phosphate, pH=6.88) was obtained from Merck. Sodium di-hydrogen phosphate, di-sodium hydrogen phosphate, sodium hydrogen carbonate and di-sodium carbonate were obtained from Merck. Deionized water was used in all experiments. Stock standard solutions of the analytes were prepared by dissolution of each drug in methanol,  having a concentration of 1000 mg L-1. Fresh standard solutions were prepared by diluting the standard solution of the analytes with deionized water of required concentration. All these solutions were stored at 4 ºC in the absence of light.

2.2. Apparatus

Chromatographic analysis was performed using a Shimadzu (LC-20AD prominence, Japan) with a photodiode array detector (SPD-M20A). Separations were carried out on a μBoundapak C18 column of 15 cm × 4.6 mm with 5 µm film thickness. HPLC data were acquired and processed using a Lab solution software (LC solution version 1.25 SP5). The mobile phase was phosphate buffer (pH 6.8)–methanol–acetonitrile (70:20:10, v/v/v) at a flow rate of 1.0 mL/min under isocratic conditions. The detection was performed at the wavelength of 284 and 308 nm for carbamazepine and lamotrigine, respectively. In the measurement of lamotrigine (LTG) and carbamazepine (CBZ) in the optimization steps and also determination in real samples, the mixture of both drugs were used. The maximum wavelengths for measurement of LTG and CBZ were 308nm and 284nm, respectively. Unfortunately, in 308nm and 284nm, both of the drugs have peaks. However, CBZ at 284nm and LTG at 308nm have peaks with high intensities. The maximum intensity of the drugs peaks at the selected wavelengths were used for the subsequent experimental.  A centrifuge model ALC 4232 was used to perform the centrifuge process (USA). The pH- meter model 827 Metrohm (Herisau, Switzerland) was used for pH measurements.

2.3. Dispersive liquid-liquid microextraction procedure

Five milliliter of sample solution containing the analytes was poured into a centrifuge glass vial. The pH of the solution was adjusted at 10 by using sodium bicarbonate. A mixture containing 30 µL of [C8MIM][PF6]  (as extraction solvent) and 100 µL of methanol (as disperser solvent) was injected into the sample solution. Cloudy solution was formed as the fine droplet of the immiscible extraction solvent dispersed in the sample. This process enlarged the contact area between the extraction solvent and sample, and the analytes were extracted into the formed fine droplets. Then it was placed in ice bath for 2 min. The cloudy solution was centrifuged at 3500 rpm for 10 min to separate the phases. Finally, 100 µL methanol was added into the collected IL and injected into the HPLC system.

3. Results and discussion

In the present study, the applicability of ionic liquid DLLME combined with HPLC was considered for the simultaneous determination of carbamazepine and lamotrigine in biological samples. There are several factors which affect the extraction process including type and volume of extracting solvent, type and volume of disperser solvent, salt concentration and pH of the sample solution. Optimization of the variables was performed using one variable at a time method. All experiments were replicated three times. The spiked concentration level in the optimization study was 5 mg L-1 of carbamazepine and 20 mg L-1 of lamotrigine. Enrichment factor (EF) and extraction recovery (ER) were calculated based on the following equations:

EF = Csedimented/C0                                                       (1)

(2)

Where, EF, Csedimented and C0 are the enrichment factor, concentration of the analytes in the sedimented phase and initial concentration of the analytes in the sample, respectively. ER%, Vsedimented and Vsample are the extraction recovery, volume of the sedimented phase and volume of the sample, respectively. Csedimented is calculated from a suitable direct injection calibration curve. Blank urine and plasma was obtained from ten different healthy volunteers. Different sources of blank urine and plasma (n=3) were used for testing the endogenous interference. There were no interfering peaks at either the carbamazepine and lamotrigine retention time.

3.1. Effect of pH

The sample pH is an important factor in the enrichment process, and which can affect the extraction efficiencies of the analytes. In this study, the pH values of the sample solutions were adjusted between 7 and 11 with buffers of sodium phosphate and sodium bicarbonate. As seen in Figure 2, the best peak areas were obtained at pH 10. The pka value for CBZ is 13.13 and the pka value for LTG is 5.3. At the pH=10; the analytes were extracted based on hydrophobic interaction. Also, in acidic pH, the structures of the drugs were decomposed. Thus, pH=10 was selected as the optimum value.

Figure 2. Effect of pH on the extraction efficiency, Extraction condition: extraction solvent: [C4MIM][PF6], extraction solvent volume: 30 µL; dispersive solvent: methanol; dispersive solvent volume: 100 µL; concentration of NaCl (w/v)%: 1.0

3.2. Selection of extraction solvent

In the selection of the extraction solvent, certain properties of the IL that need to be considered are: (1) to extract carbamazepine and lamotrigine well, (2) to have higher density than water, and (3) to form a cloudy solution in the presence of dispersive solvent. In this study, three ionic liquids, including [C8MIM][PF6], [C6MIM][PF6] and [C4MIM][PF6] were investigated. By comparing them as extraction solvent, it was observed that carbamazepine and lamotrigine exhibited a better affinity for [C8MIM][PF6], because of more solubility of the mentioned drugs in [C8MIM][PF6] (Figure 3). Therefore, [C8MIM][PF6] was selected as extraction solvent in the subsequent experiments.

Figure 3. Effect of type of extraction solvent on the extraction efficiency, Extraction condition: extraction solvent volume: 30 µL; dispersive solvent: methanol; dispersive solvent volume: 100 µL; concentration of NaCl (w/v)%: 1.0, pH:10

3.3. Effect of extraction solvent volume

Optimization of the volume of the IL as an extraction solvent is a further step in development of a IL-DLLME procedure. The volume of the extraction solvent can influence formation of dispersion and thus have to be optimized. In order to study the effect of extraction solvent volume, different volumes of [C8MIM][PF6] (20–50 µL in 10 µL intervals) were tested. It was observed (Figure 4) that the peak areas were increased by increasing the [C8MIM][PF6] volume up to 30 µL for carbamazepine and lamotrigine. The peak areas of the analytes decreased by increasing the volume of [C8MIM][PF6], which was an expected result due to dilution of the extracted analytes in the extraction solvent at higher volumes. Therefore, 30 µL of [C8MIM][PF6] was selected as the optimum volume.

Figure 4. Effect of volume of extraction solvent on the extraction efficiency, Extraction condition: extraction solvent: [C8MIM][PF6], dispersive solvent: methanol; dispersive solvent volume: 100 µL; concentration of NaCl (w/v)%: 1.0, pH:10

3.4. Effect of type of disperser solvent

In the IL- DLLME, the disperser solvent should be soluble in the extraction solvent and miscible in the sample solution, thus, enabling the formation of fine droplets of the extraction solvent. Therefore, acetonitrile, methanol, and acetone were tested as dispersive solvents. A series of sample solutions were examined using 100 µL of each of the disperser solvents containing 30 µL of [C8MIM][PF6]. By using acetone and acetonitrile, the cloudy solution do not formed well.  It was clear that (Figure 5), the best peak areas was obtained when methanol was used as a disperser solvent. Hence, the subsequent experiments were performed using methanol as the disperser solvent.

Figure 5. Effect of type of disperser solvent on the extraction efficiency, Extraction condition: extraction solvent: [C8MIM][PF6], extraction solvent volume: 30 µL; dispersive solvent volume: 100 µL; concentration of NaCl (w/v)%: 1.0, pH:10

3.5. Effect of volume of disperser solvent

In order to study the effect of disperser solvent volume, different volumes of methanol (50, 100, 300, 500 and 1000 µL) were used. It is clear from figure 6 that 100 µL methanol had the highest peak areas. It seems that, at the volume of 100 µL, the amount of methanol was enough for effective forming of the cloudy solution. At a low volume of methanol, cloudy solution was not properly formed resulting in a decrease in the peak areas.  At a high volume of methanol, the solubility of the analytes in the sample increased resulting in a decrease in the peak areas.  Thus, 100 µL was selected as the optimum volume of methanol.

Figure 6. Effect of volume of methanol on the extraction efficiency, Extraction condition: extraction solvent: [C8MIM][PF6], extraction solvent volume: 30 µL; dispersive solvent: methanol; concentration of NaCl (w/v)%: 1; pH: 10

3.6. Salt addition

Generally, salt addition can cause a decrease in the solubility of the analytes in sample solution and enhance extraction efficiency. To evaluate the possibility of salting out effect, the extraction efficiency was studied with the sodium chloride ranging from 0.5 to 1.5 (w/v) % (Figure 7). Due to the salting-out effect, the peak areas increased as the amount of NaCl increased from 0.5 to 1.0 (w/v) %. By increasing the ionic strength (from 1.0 to 1.5 (w/v) %), a reduction of the peak areas for carbamazepine and lamotrigine were observed, because of dilution effect. Based on the results, 1.0 (w/v)% of NaCl was added in all the subsequent experiments.

Figure 7. Effect of NaCl concentration on the extraction efficiency, Extraction condition: extraction solvent: [C8MIM][PF6], extraction solvent volume: 30 µL; dispersive solvent: methanol; dispersive solvent volume: 100 µL; pH: 10

3.7. Analytical performance and method validation

Calibration data of carbamazepine and lamotrigine were obtained using IL-DLLME-HPLC system under optimum conditions. For urine samples, the linearity of calibration curve was observed in the range of 0.07-20 mg L-1 for carbamazepine and 0.17-40 mg L-1 for lamotrigine. The coefficients of determination (r2) were 0.991 and 0.997 for carbamazepine and lamotrigine, respectively. The limits of detection (LODs) based on signal-to-noise ratio (S/N) of 3, were 0.02 and 0.05 mg L-1 for carbamazepine and lamotrigine, respectively. The limit of quantifications (LOQs), based on signal-to-noise ratio (S/N) of 10, were 0.07 and 0.17 mg L-1 for carbamazepine and lamotrigine, respectively . The relative standard deviation (RSD%, n=5) at the concentration level of 5.0 mg L-1 of carbamazepine and lamotrigine were 1.7% and 5.6% for carbamazepine and lamotrigine, respectively. Enrichment factors were 35 and 26 for carbamazepine and lamotrigine, respectively. Extraction recoveries were 70 and 52% for carbamazepine and lamotrigine, respectively. However, for plasma samples, linearity was observed in the range of 0.1-20 mg L-1 for  carbamazepine and 0.3-40 mg L-1 for lamotrigine. The r2 were 0.987 and 0.995 for carbamazepine and lamotrigine, respectively. The limits of detection (LODs) based on signal-to-noise ratio (S/N) of 3, were 0.04 and 0.07 mg L-1 for carbamazepine and lamotrigine, respectively. The limit of quantifications (LOQs), based on signal-to-noise ratio (S/N) of 10, were 0.1 and 0.3 mg L-1 for carbamazepine and lamotrigine, respectively. The relative standard deviation (RSD%, n=5) at the concentration level of 5.0 mg L-1 of carbamazepine and lamotrigine were 3.2% and 8.4% for carbamazepine and lamotrigine, respectively. Enrichment factors were 27 and 19 for carbamazepine and lamotrigine, respectively. Extraction recoveries were 54 and 38% for carbamazepine and lamotrigine, respectively.

The selectivity of the method was evaluated by analysing six blank plasma and urine samples to evaluate the existence of matrix endogenous substances in retention times that could interfere with carbamazepine (CBZ) and lamotrigine (LTG). The analysis of blank human plasma and urine samples from six healthy volunteers confirmed the absence of endogenous interferences in the retention times of carbamazepine and lamotrigine.

The stability of CBZ and LTG stock solutions were evaluated at room temperature for 8 hours and 24 hours and after storage at −20°C for 10 days. Stability was calculated by comparing the pertinent responses obtained from the tested stock solution(s) with the responses of freshly prepared ones and the result are given in Table 1. According to the results obtained, CBZ and LTG was stable in human plasma and urine samples in the different storage conditions.

Table 1: Summary of stability of CBZ and LTG in stock solution and human plasma and human urine

| Data of Stock Solution Stability |  |  |  |
| --- | --- | --- | --- |
| Drug(n=5) | 8 hrs at RT | 24 hrs at RT | 10 Days at −20°C |
| CBZ |  |  |  |
| Precision (%) | 1.2 | 1.8 | 1.5 |
| Accuracy (%) | 100.1 | 99.8 | 98.7 |
| LTG |  |  |  |
| Precision (%) | 3.5 | 3.7 | 3.0 |
| Accuracy (%) | 99.6 | 99.3 | 98.9 |
| Data of Stability in Plasma Samples |  |  |  |
| CBZ |  |  |  |
| Precision (%) | 3.0 | 3.3 | 3.6 |
| Accuracy (%) | 98.1 | 97.9 | 97.2 |
| LTG |  |  |  |
| Precision (%) | 7.8 | 8.0 | 7.6 |
| Accuracy (%) | 98.3 | 97.8 | 98.1 |
| Data of Stability in Urine Samples |  |  |  |
| CBZ |  |  |  |
| Precision (%) | 1.5 | 1.4 | 1.6 |
| Accuracy (%) | 98.8 | 99.1 | 97.8 |
| LTG |  |  |  |
| Precision (%) | 4.8 | 5.1 | 5.0 |
| Accuracy (%) | 98.2 | 98.5 | 97.8 |

Absolute recovery of the analytes were determined in triplicates at high, medium and low

concentrations in plasma and urine by extracting drug free plasma and urine samples spiked with CBZ and LTG. Recovery was calculated by comparison of the analyte peak-areas of the extracted samples with those of the unextracted analyte standards, representing 94 and 88 % recovery of CBZ and LTG respectively in plasma and 99 and 94 % recovery of CBZ and LTG respectively in urine .

In order to evaluate the effect of matrix samples on the performance of the proposed method, determination of CBZ and LTG in human urine and plasma samples at the three different concentration levels were performed. For doing the IL-DLLME procedure on the plasma samples, some extra processes are needed. First the human plasmas were dissolved in a suitable amount of acetonitrile such as 1:1 (V/V) reducing the matrix effect and then the mixtures were centrifuged. Secondly, they were filtered for getting a clear solution and removing the dirty solution at the bottom of test tubes. The samples was found to be free from the drugs. Therefore, specific amounts of CBZ and LTG at the three different concentration levels were spiked to the samples and analyzed by the proposed method. The spiking recoveries of the target compounds in the urine and plasma samples are summarized in Table 2. The relative recovery (RR) is obtained from the following equation:

RR% = Cfound - Creal /Cadded × 100,

where Cfound, Creal and Cadded are the concentrations of the analytes after the addition of a known amount of standard in a real sample, the concentration of the analytes in a real sample and the concentration of a known amount of standard, which was spiked to the real sample, respectively. The relative recoveries were between 87-103% (Table 2) and showed that the matrix had negligible effect on the performance of the proposed method. The chromatograms of the urine and plasma sample (without spiked and with spiked) are shown in Figures 8 and 9, respectively.

Table 2

Determination of carbamazepine (CBZ) and lamotrigine (LTG) in human plasma and urine by IL-DLLME-HPLC-DAD Sample

|  | Spiked concentration (mg L-1) |  |  |  |  |  | Relative recovery% ± SD (n=3)a |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CBZ |  |  | LTG |  |  | CBZ |  |  | LTG |  |  |
| Human urine | 2 | 4 | 10 | 5 | 14 | 20 | 93.0 ± 3.1 | 103.0 ± 1.5 | 101.0 ± 1.2 | 90.0 ± 5.0 | 97.0 ± 3.5 | 99.0 ± 4.1 |
| Human plasma | 2 | 4 | 10 | 5 | 14 | 20 | 89.0 ± 5.4 | 94.0 ± 2.8 | 96.0 ± 2.5 | 87.0 ± 9.5 | 91.0 ± 4.5 | 93.0 ± 3.1 |

aStandard deviation

Figure 8. HPLC chromatograms, (a,b) before spiking with analytes in urine at the wavelength of 308 and 284 nm for lamotrigine and carbamazepine, respectively, (c) 14 (lamotrigine) and (d) 4 (carbamazepine) mg L-1 spiked of analytes in urine after extraction via proposed method at optimum conditions. (The retention time of LTG was 5.1 min and the retention time of CBZ was 12.1 min at the measurement in the maximum wavelengths).

Figure 9. HPLC chromatograms, (a,b) before spiking with analytes in plasma at the wavelength of 308 and 284 nm for lamotrigine and carbamazepine, respectively,, (c) 14 (lamotrigine) and (d) 4 (carbamazepine) mg L-1 spiked of analytes in plasma after extraction via proposed method at optimum conditions. (The retention time of  LTG was 5.1 min and the retention time of CBZ was 12.1 min at the measurement in the maximum wavelengths).

Table 3 compare the proposed method with the other extraction methods for the determination of the target analytes in biological samples. The comparison of extraction time of the proposed method with solid-phase microextraction (SPME),32 liquid-liquid extraction (LLE)33,34 and solid-phase extraction (SPE)35 for the extraction of the target analytes indicates that this novel method has a very short equilibrium time comparing to the mentioned methods and the extraction time needed for the proposed method is a few seconds. Quantitative results of the proposed method is better than SPE35 and LLE33,34 methods. Relative standard deviation (RSD%) of the proposed method is better than SPME 32 and LLE33methods. Also, SPE and LLE methods are time-consuming and laborious, and the large amounts of organic solvents used in the extraction procedures cause problems with regard to health and the environment. Finally, the extraction solvent used in DLLME generally is highly toxic and not environmentally friendly. Ionic liquids (ILs) are considered to be “environmental friendly solvents”. In the proposed work, in DLLME method, IL as extraction solvent was used.

Table 3

Comparison of the proposed method with other extraction methods for the determination of carbamazepine (CBZ) and lamotrigine (LTG)

| Methods | Sample | R.S.D.% | Dynamic linear range (mg L-1) | Limit of detection (mg L-1) | Extraction time (min) | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
| SPME-GC-TSD | Plasma | <10 | 0.06-20 (CBZ); 0.2-10 (LTG) | 0.06 (CBZ); 0.2 (LTG) (Limit of quantitation) | 15 | [32] |
| Precipitation and liquid extraction-GC-MS | Serum | <12 | 0.625-20 (CBZ, LTG) | - | 5 | [33] |
| LLE-HPLC-UV | Plasma | <6 | 1.0-30 (LTG) | 0.15 (LTG) | 5 | [34] |
| SPE-HPLC-DAD | Plasma | <8 | 0.2-25 (CBZ) | 0.02 (CBZ) | 1 | [35] |
| IL-DLLME-HPLC-DAD | Urine and plasma | Urine (1.7 (CBZ), 5.6 (LTG) Plasma (3.2 (CBZ), 8.4 (LTG) | Urine (0.07-20 (CBZ), 0.17-40 (LTG) Plasma (0.1-20 (CBZ), 0.3-40 (LTG) | Urine (0.02 (CBZ), 0.05 (LTG) Plasma (0.04 (CBZ), 0.07 (LTG) | A few seconds | This work |

4. Conclusion

A rapid and simple method using the ionic liquid-based dispersive liquid–liquid microextraction procedure was presented to the extract and concentrate carbamazepine and lamotrigine from biological samples. The proposed microextraction method is environmentally friendly (highly toxic chlorinated solvents are not required), rapid and simple set up. The proposed method has satisfied LODs and precisions which were in the range of 0.02-0.07 mg L-1, precisions were in the range of 1.7-8.4%. The proposed method was also applied for the analysis of drugs in urine and plasma samples and the spiked recoveries were in the range of 87-103% . All these results indicated that the proposed method had advantages such as good sensitivity, simplicity, easy to operate, limited chance of exposure to the toxic solvent and high enrichment factor.  This study provides a new perspective regarding the replacement of chlorinated solvents with less-toxic solvents in DLLME and supports the use of green analytical chemistry methods. In the final experiment, the developed method was applied to the determination of carbamazepine and lamotrigine in biological samples and the acceptable results can be achieved.

Acknowledgement

Financial support by Lahijan Branch, Islamic Azad University (Lahijan, Iran) during the period of this research is gratefully acknowledged.

Compliance with Ethical Standards

Funding There is no funding for this study.

Conflict of Interest No conflict exists; author Ameneh porgham Daryasari declares that she has no conflict of interest. Author Salumeh Ranjbar declares that she has no conflict of interest. Author Mojtaba soleimani declares that he has no conflict of interest.

Ethical approval This article does not contain any studies with human participants or animals performed by any of the authors.

Informed consent No humans are involved in this study.

References

1. H.M. Neels, A.C. Sierens, K. Naelaerts, S.L. Scharpe, G.M. Hatfield, W.E. Lambert, Clin. Chem. Lab. Med. 2004, 42, 1228–1255. DOI: 10.1515/CCLM.2004.245

2. A. Fortuna, J. Sousa, G. Alves, A. Falcao, P. Soares-da-Silva, Anal. Bioanal. Chem. 2010, 397, 1605–1615. DOI:10.1007/s00216-010-3673-0

3. A.G. Gilman, J.G. Hardman, L.E. Limbird, Goodman and Gilman’s the Pharmacological Basis of Therapeutics, 10th edn (McGraw Hill, New York, USA), 539, 2001.

4. S.C. Sweetman, Martindale, The Complete Drug Reference, 34th edn (Pharmaceutical Press, London), 363, 2005.

5. M. Silanpaa, L. Haataja, T. Tomson, S.I. Johannessen, Carbamazepine, in: S. Shorvon, E. Perucca, J. Engel (Eds), The treatment of epilepsy, third ed. Blackwell Publishing LTD., Oxford, 2009, pp. 459-474.

6. M. Sigrid, F. Ricardo, V. Mario, D. Marta, C. Godoy, JPC-J. Planar Chromat. 2011, 24, 222-226. DOI: 10.1556/JPC.24.2011.3.9

7. N. Jebabli, E. Gaies, H. Jebari, R. Charfi, M. Lakhal, A. Klouz, S. Trabelsi, I. Salouage, Tunis Med. 2015, 93, 565-568. 8. N.F. Youssef, E.A. Taha, Chem. Pharm. Bull. 2007, 55, 541-545. DOI: 10.1248/cpb.55.541

9. P. Nikolaou, I. Papoutsis, A. Dona, C. Spiliopoulou, S. Athanaselis, J. Pharm. Biomed. Anal. 2015, 102, 25-32. DOI: j.jpba.2014.08.034

10. G.S. Elizabeth, S. Giannoutsos, D.R. Lower, M.A. Virji, M.D. Krasowski, J. Chromatogr. Sci. 2007, 45, 616-622. DOI: 10.1093/chromsci/45.9.616

11. R. Theurillat, M. Kuhn, W. Thormann, J. Chromatogr. A 2002, 979, 353-368. DOI: 10.1016/s0021-9673(02)01257-8

12. P.E. Morgan, D.S. Fisher, R. Evers, R. J. Flanagan, Biomed. Chromatogr. 2011, 25, 775-778. DOI: 10.1002/bmc.1515

13. S. Ghatol, V. Vithlani, S. Gurule, A. Khuroo, T. Monif, P. Partani, J. Pharm. Anal. 2013, 3, 75-83. DOI: https://doi.org/10.1016/j.jpha.2012.09.001

14. C. Linder, A. Hansson, S. Sadek, L.L. Gustafsson, A. Pohanka, J. Chromatogr. B 2018, 1072, 116-122. DOI: https://doi.org/10.1016/j.jchromb.2017.11.005

15. J.M. Juenke, K.A. Miller, M.A. Ford, G.A. Mcmillin, K.L. Johnson-Davis, Clin. Chim. Acta 2011, 412, 1879-1882. DOI: https://doi.org/10.1016/j.cca.2011.06.009

16. L.A. Romanlyshyn, J.K. Wichmann, N. Kucharczyk, R.C. Shumaker, D. Ward, R.D. Sofia, Ther. Drug Monit. 1994, 16, 90-99.

17. K. He, T. Yonetani, Y. Asada, S. Echigo, S. Itoh, Mirochem. J. 2019, 145, 1191-1198. DOI: https://doi.org/10.1016/j.microc.2018.12.014

18. A. Klancar , M. Zakotnik, R. Roskar, J. Trontelj, Anal. Methods 2017, 9, 5310-5321. DOI: 10.1039/C7AY01310H

19. M.D. Cantu, D.R. Toso, C.A. Lacerda, F.M. Lancas, E. Carrilho, M.E. Queiroz, Anal. Bioanal. Chem. 2006, 386, 256–263. DOI:10.1007/s00216-006-0629-5

20. S. Bompadre, A. Tagliabracci, M. Battino, R. Giorgetti, J. Chromatogr. B Analyt. Technol. Biomed. Life Sci. 2008, 863, 177–180. DOI:10.1016/j.jchromb.2007.12.020

21. W. Lee, J.H. Kim, H.S. Kim, O.H. Kwon, B.I. Lee, K. Heo, Neurol. Sci. 2010, 31, 717–720. DOI:10.1007/s10072-010-0257-x

22. N.R. Barbosa, A.F. Midio, J. Chromatogr. B Biomed. Sci. Appl. 2000, 741, 289–293. DOI: 10.1016/S0378-4347(00)00102-X

23. A. Ferreira, M. Rodrigues, P. Oliveira, J. Francisco, A. Fortuna, L. Rosado, P. Rosado, A. Falcao, G. Alves, J. Chromatogr. B Analyt. Technol. Biomed. Life Sci. 2014, 971, 20–29. DOI:10.1016/j.jchromb.2014.09.010

24. M. Rezaee, Y. Assadi, M.R.M. Hosseini, E. Aghaee, F. Ahmadi, S. Berijani, J. Chromatogr. A 2006, 1116, 1–9. DOI:10.1016/j.chroma.2006.03.007

25. R. Rahnama, Z.C. Jojadeh, M.R.  Jamali, Acta Chim. Slov. 2012, 59, 641-647.

26. M.R. Hadjmohammadi, S.S. Ghoreishi, Acta Chim. Slov. 2011, 58, 765-771.

27. S. Shariati, M. Golshekan, Acta Chim. Slov. 2011, 58, 311-317.

28. P. Anastas, N. Eghbali, Chem. Soc. Rev. 2010, 39, 301–312. DOI:10.1039/b918763b

29. S. Xiulan, Z. Li, Y. Fang, P. Chen, G. Ren, H. Shan, Curr. Anal. Chem. 2010, 6, 249–259. DOI: 10.2174/157341110791516954

30. J.F. Liu, J.A. Jِnsson, G.B. Jiang, Trends Anal. Chem. 2005, 24, 20–27. DOI: 10.1016/j.trac.2004.09.005

31. R. Rahnama, N. Mansoursamaei, M.R. Jamali, Acta Chim. Slov. 2014, 61, 191-196.

32. M.E.C. Queiroz, S.M. Silva, D. Carvalho, J. Chromatogr. Sci. 2002, 40, 219-223. DOI:

10.1093/chromsci/40.4.219

33. J. Hallbach, H. Vogel, W.G. Guder, Eur. J. Clin. Chem. Clin. Biochem. 1997, 35, 755-759. DOI: 10.1515/cclm.1997.35.10.755

34. E. Greiner-Sosanko, S. Giannoutsos, D.R. Lower, M.A. Virji, M.D. Krasowski, J. Chromatogr. Sci. 2007, 45, 616-622. DOI:10.1093/chromsci/45.9.616

35. P. Dzodic, L. Zivanovic, A. Protic, I. Ivanovic, R. Velickovic-Radovanovic, M. Spasic, S. Lukic, J. Serb. Chem. Soc. 2012, 77, 1423–1436. DOI: 10.2298/JSC120106084D