Exergy Analysis of a Turbo Expander: modeling and simulation

Abstract

Natural gas is a mixture that is widely used in the industries. Knowledge of its thermodynamic properties is essential for evaluating the process and equipment performance.

This paper quantifies the energy that can be extracted from natural gas using a turbo expander. Natural gases of wide-ranging compositions collected from 6 different gas fields in Egypt were investigated based on energy and exergy analysis. The study was conducted using MATLAB. Numerous simulation runs were made by taking various typical feed compositions classified as lean and rich.

The effects of increasing the amount of C1, C5 in the feed stream on the efficiency of energy utilization are presented. A validation analysis was performed. The results show similar trends and good agreements. It was concluded from the results that when the concentration of methane in the gas mixture increase, the exergetic efficiency decreases. The results also show that the values of thermodynamic properties depend on the relative amount of heavy components in the feed stream.

Keywords: exergy efficiency, turbo-expander, Peng Robinson, Feed composition effect, Natural gas

Introduction

Cryogenic separation methods have been widely used in the petrochemical industry. The main disadvantage of cryogenic methods is it requires large amounts of refrigeration. The search for new methods to accomplish this is of great significance in the industry. 1-3 The interest in low-grade heat recovery has increased in the past decades. 4-6 A significant amount of exergy is destroyed in the transmission and distribution of natural gas as a result of a passive gas expansion process (Joule–Thomson) that brings the pressure from production sites (high transmission levels) to load centers (lower pressures). 7-8 A turboexpander can be used to replace a throttle valve in the expansion process to generate mechanical work or electrical power.9 It was found that the replacement of a throttling valve with an expander can increase the exergy efficiency by 30% . 10

The power extracted by turbo-expanders can deliver large amounts of energy through electrical generators, compressors and other loads.11 The term "Turbo-expander" is normally used to define an Expander/Compressor machine as a single unit. Turbo-expander is a centrifugal or axial flow turbine through which high-pressure gas is expanded to produce work that is often used to drive a compressor side. 12 Turboexpander units are often used for the recovery of heavier hydrocarbons from natural gas.13-14 However, the introduction of a turbo-expander in gas processing industries has certain limitations in terms of operational flexibility and overall recovery performance. 15

Numerical simulation by computational modeling is a remarkable alternative technique in the investigation of the performance of many alternative energy sources. 16 Some investigation on turbo-expander modeling was introduced for energy recovery in electrical form and to study the mechanical oscillation of the shaft and power quality issues of the generated power. 17-18

The feed composition is a very vital factor that affects NGL recovery. Since the feed normally originates from upstream plants, there is a continuous variation of feed composition which increases the instability of the plant. There also exists a seasonal variation of the feed composition, with leaner feed gas, mostly occurring during cooler months and richer gas in the warmer months.

In this work, the effect of different feed characteristics on exergy efficiency was investigated; various feed compositions were considered and characterized under the basic classifications of lean and rich feed. A numerical method and a computer program were used to calculate the thermal properties of natural gas mixture such as enthalpy using Peng Robinson state equation. Matlab has been utilized to model turbo-expander. The exergy analysis of a turbo-expander unit and effect of different feed compositions has been investigated and validated according to the first and second laws of thermodynamics, and the performance of the TX is evaluated according to a different composition.

- Mathematical Model Development

A model of the proposed unit has been created to determine the performance using real site data. Data for inlet pressure, temperature, flow rate, and required outlet pressure are key inputs.

The model will then calculate the rate of power and system efficiency. Numerical modeling of the exergy recovery is performed using MATLAB.

To simplify the theoretical model of the turbo-expander, the following assumptions are made:

- There are no pressure losses in pipes and heat exchangers.

- The small difference between the intermediate pressure, the discharge pressure of the low-stage compressor and the suction pressure of the high-stage compressor is negligible.

- The NG is dry; no gas hydrates are formed.

- The gearbox and generator efficiencies are assumed to be constant.

The main input was the inlet temperature, which was used for the calculation of the inlet pressure by using a Peng Robinson equation. Peng Robinson equation of state is a semi-empirical equation of state that is derived from the Van der Waals equation of state, 19-22 which is as follows:

Pin=    R*TinVm-b- a   Vm2 +2*b*Vm-b2        Eq. (1)

Where

a =R2* Tpc22.1870 * Ppc  (1+K{1-Tre12})2

b=R*Tpc12.8535*Ppc

K  =0.37464+1.54226*ω+ 0.26922*ω2

After that the thermodynamic fluid properties were determined, the correlations of Farazneh-Gord & Rahbari, 23 were used. The correlations are based on measurable real-time properties such as Temperature, Pressure, and specific gravity (molecular weight) of the natural gas for the non-measurable thermodynamic properties such as Entropy, Enthalpy, and Internal energy.

hi(T’, P’,ϒ’) = A1(T’,ϒ’) P’4 + A2(T’,ϒ’) P’3 + A3(T’,ϒ’) P’2 + A4(T’, ϒ’) P’+ A5(T’, ϒ’)                                                                                  Eq. (2)

Where

Ai (T’, ϒ’); i=1,…, 5 are defined as:

Ai (T’,ϒ’) = B1 (T’) ϒ’2 + B2 (T’) ϒ’ + B3 (T’)

And Bj (T); j=1, 2, 3 for each Ai (T’, ϒ’) are defined as:

Bj(T’)=C1T’2 – C2 T’ –C3

Where: T’, P’, and ϒ’ are functions of T, P, and gas specific gravity respectively as follows:

Ti’ = Tin - 30050,  Pi’ = Pin - 137.3598     , ϒi= Mwt28.966   ,      ϒi’ = ϒi - 0.625410.07894

Exergy analysis is based on the first and second laws of thermodynamics and is employed to calculate the maximum useful power accessible by an assigned amount of input energy to a process. By neglecting the effects of kinetic, potential and nuclear energies (unchanged or insignificant), the total exergy can be divided into two parts, chemical, and physical exergy. Eqs. (3) – (5) express the exergy destruction, the total exergy, and the physical exergy, respectively

E˙ destroyed =ToS˙                                                  Eq. (3)

E˙= E˙ph   + E˙chem	Eq. (4)

E˙ph  = (H˙-H˙o ) – T o( S˙- S˙o)	Eq. (5)

The outlet enthalpy is used in conjunction with the isentropic enthalpy and inlet enthalpy to determine isentropic expansion efficiency. The working fluid enters the turbo-expander and expands to the discharge pressure with the turbo-expander efficiency is defined as:

ηE=hi - hohi - ho,s,e                                                      Eq. (6)

The electrical power obtained from the exergy recovery process can be expressed as:

W .Gen = m .N.G (hi – ho) .ηGB.ηGe                                           Eq. (7)

The exergetic efficiency of the recovery can be obtained by comparing the reversible expansion work to the actual work produced by the electric generator. Therefore, by substituting in the following form:

ηII,P= w’Genw’Rev=(hi – ho) . ηGB. ηGen{hi – ho,s,e- To (Si-So,s,e)}                        Eq. (8)

Table 1: Typical Feed Selected to Study

| Composition (Mole %) |  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Borapetco | PS | PS | Borapetco | AQP | AQP |
|  |  | El Salmiya- 2 | AES E2 (TAMMAM) | AES-E6#1/1 | ZZ-4X | W-A/Q (Sep. Gas) | A/Q (Feed Gas) |
| N2 | N2 | 0.95 | 0.905 | 0.617 | 0.059 | 0.1108 | 0.0864 |
| CO2 | CO2 | 3.914 | 0.426 | 0.518 | 1.771 | 0.8264 | 0.6117 |
| H2S | H2S | 0 | 0 | 0 | 0 | 0 | 0 |
| C1 | C1 | 63.807 | 78.915 | 79.888 | 80.53 | 88.857 | 93.375 |
| C2 | C2 | 13.129 | 8.97 | 9.151 | 8.825 | 4.9658 | 3.2473 |
| C3 | C3 | 8.656 | 5.483 | 4.898 | 4.135 | 2.3345 | 1.3393 |
| iC4 | C4 | 4.808 | 3.045 | 2.854 | 1.919 | 1.2878 | 0.7635 |
| nC4 |  |  |  |  |  |  |  |
| iC5 | C5 | 1.924 | 1.308 | 1.318 | 1.026 | 0.3526 | 0.2715 |
| nC5 |  |  |  |  |  |  |  |
| C6+ | C6+ | 2.812 | 0.948 | 0.756 | 1.735 | 1.2655 | 0.3051 |

To study the different feed composition types; six typical feed samples were selected from six fields of six reservoirs shown in table 1 and the operating conditions are shown in table 2.

Table 2 feed operating conditions

| Well # | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Company | Borapetco | PS | PS | Borapetco | AQP | AQP |
| Well Name | El Salmiya-2 | AES E2 (TAMMAM) | AES-E6#1/1 | ZZ-4X | W-A/Q (Sep. Gas) | A/Q (Feed Gas) |
| Production Rate (MMSCFD) | 15 | 25 | 20 | 15 | 50 | 100 |
| Well Life in the Declining Production Regime (Years) | 13 (2012-2025) | 20 (2009-2029) | 15 (2013-2033) | 17 (2012-2029) | 25 (2006-2031) | 30 (2000-2030) |
| Feed Pretreatment for Transportation & its Conditions | Corrosion Inhibitor Injection | Corrosion Inhibitor Injection | Corrosion Inhibitor Injection | Corrosion Inhibitor Injection | Corrosion Inhibitor Injection | Corrosion Inhibitor Injection |
| The Design of a Turbo-Expander & its Operating Modes |  |  |  |  |  |  |
| Max. F (MMSCFD) Design P (bar g) Max. Design T (˚C) Min Design T (˚C) Operating Mode | 50 79 80 -35 Isentropic | 75 60 80 -20 Isentropic | 75 60 80 -20 Isentropic | 50 79 80 -35 Isentropic | 120 115 58 -70 Isentropic | 120 115 58 -70 Isentropic |
| Mechanical Efficiency Electrical Efficiency | 75 % 70 % | 80 % 75 % | 80 % 75 % | 75 % 70 % | 84 % 80 % | 84% 80 % |
| The Target Substances of Petrochemical Synthesis & Engine Fuels Produced from the HC’s. | N.G (fuel) | N.G (fuel) | N.G (fuel) | N.G (fuel) | LPG (fuel) | LPG (fuel) |

- Results and discussion:

Because of the natural gas composition variation in the selected samples is the one who plays the main role in the study, and also methane content represents the huge fraction of that composition. Therefore, the impact of the methane concentration variation on the selected samples of the thermodynamic properties was studied and validated using Aspen HYSYS V10. By entering the different feed compositions and operating condition; the hysys program calculates the thermodynamic properties of the different streams from which expander and exergetic efficiency can be predicted.

3.1. Studying gas composition variation effect on enthalpy:

In this case, the selected feed samples are arranged in increasing order of hydrocarbon contents, from the C1 content to heavier hydrocarbon.

Also, pentane content and its variations in the natural gas composition have been chosen as a basis to show its effect as a heavy component on the previous mentioned thermodynamic properties. In this case, the selected feed samples are re-arranged in decreasing order of hydrocarbon contents, from C5 content to lighter hydrocarbon.

The first four feed samples (1-4) are classified as rich feed, while the remaining two feed samples (5&6) are taken as lean feed.

The classification for rich feed is merely based on the contents of C2 and/or C3. If the C2 content is less than 8% or the C3 content less than 3% or the C2 plus C3 contents less than 10%, the feed is considered a lean feed; otherwise, it is taken as a rich feed. 14

3.1.1. Effect of methane concentration variation on inlet enthalpy:

Fig.1: Effect of methane concentration variation on inlet enthalpy (kJ/kg)

Figure (1) shows the relation between methane concentration (mole %) and the inlet enthalpy (kJ/kg).

To sense their effects on each other, the inlet conditions (pressure and temperature) for all gas samples were kept the same as typical actual plant conditions at 66.2 barg and 21.9 °C respectively.

The difference in the feed enthalpy for the six feed types arises from the variation of the feed compositions. The relationship between the variation in methane concentration and the inlet enthalpy of the gas mixture is a close linear relationship in an inverse proportion.

3.1.2. Effect of pentane concentration variation on inlet enthalpy:

Fig.2 : Effect of pentane concentration variation on inlet enthalpy (kJ/kg)

As illustrated in figure (2) shows the relationship between the variation in pentane concentration and the inlet enthalpy of the gas mixture is a quasi-linear direct proportional relationship. It is observed that the leaner feed generally give lower duty than those of rich feed. This is due to the presence of high heavier hydrocarbons (C5+) components in the rich feed.

The linearity deviation is almost due to an erratic point. The point is 1.924 C5 mole %, which is the lowest sample in C1 content “El Salmiya-2, Borapetco”, therefore the enthalpy is decreased.

3.1.3. Effect of methane concentration variation on outlet enthalpy:

(a) Using Constant Operating Conditions:

Fig.3: Effect of methane concentration variation on outlet enthalpy (kJ/kg), Constant outlet conditions

Figure (3) shows the relation between the variation in methane concentration (mole %) and the outlet enthalpy (kJ/kg) in case of unification of the outlet conditions (pressure and temperature) at 26 barg and -2.2 °C as follows:

The methane rejection (The ratio of the molar flow of methane “residue” to the molar flow of methane “natural-gas feed”) is directly related to the feed-stream temperature on which the product temperature depends on. Rejection increases with decreasing temperature; however, in Konukman et.al (2005), 24 the methane rejection was found to be not very sensitive to the feed-stream temperature and in turn to the product temperature, and because outlet enthalpy depends on the temperature at a given pressure. So it is concluded that increasing methane % will not affect outlet enthalpy.

(b) Using Variable Operating Conditions:

To study the effect of changing the operating conditions on the variation in the outlet enthalpy, the plant real data were used for that by taking the required turbine duty to expand a certain gas sample from the utilized typical feed samples in this work, which corresponds to approximately 40 bar pressure drops and taking this value as a reference and set for all other samples.

Fig.4: Effect of methane concentration variation on outlet enthalpy (kJ/kg) and outlet temperature (°C), Variable outlet conditions

Consequently, it has resulted in a change in outlet conditions (pressure and temperature). Therefore, the changes in the outlet enthalpy for all samples were obtained and recorded in figure (4).

The relatively high linearity deviation is almost due to some erratic points. Mainly these points of the rich samples, and for this reason, the enthalpy is raised.

3.1.4. Effect of pentane concentration variation on outlet enthalpy:

Fig.5: Effect of pentane concentration variation on outlet enthalpy (kJ/kg) and outlet temperature (°C), Variable outlet conditions

Figure (5) shows the relationship between the variation in pentane concentration, and the inlet enthalpy of the gas mixture is a non-linear direct proportional relationship. It is observed that the leaner feed generally give lower duty than those of rich feed. This is due to the presence of high heavier hydrocarbons (C5+) components in the rich feed.

    - Effect of methane concentration variation on the isentropic outlet enthalpy:

Fig.6: Effect of methane concentration variation on the isentropic outlet enthalpy (kJ/kg) and outlet temperature (°C)

From figure (6); the relationship between the variation in methane concentration and the outlet enthalpy of the gas mixture is a quasi-linear inverse proportional relationship, i.e., when the concentration of methane in the gas mixture increases, the outlet enthalpy decreases.

    - Effect of pentane concentration variation on the isentropic outlet enthalpy:

Fig.7: Effect of pentane concentration variation on the isentropic outlet enthalpy (kJ/kg and isentropic outlet temperature (°C)

As shown from the figure (7), the relationship between the variation in pentane concentration and the isentropic outlet enthalpy of the gas mixture is a quasi-linear direct proportional relationship, as shown before that the leaner feed generally give lower duty than those of rich feed. This is due to the presence of high heavier hydrocarbons (C5+) components in the rich feed.

3.2. Effect of feed composition on entropy

3.2.1. Effect of methane concentration variation on inlet entropy:

Fig.8: Effect of methane concentration variation on the inlet entropy (kJ/kg.K)

Figure (8) shows the relationship between the variation in methane concentration, and the inlet entropy of the gas mixture is a closely linear relationship in an inverse proportion. The results are consistent with the one observed in the discussion of the relation between methane concentration and the inlet enthalpy. As known, the value of enthalpy is proportional to the values of entropy.

3.2.2. Effect of pentane concentration variation on the inlet entropy:

Fig.9 Effect of pentane concentration variation on the inlet entropy (kJ/kg.K)

From the illustrated figure (9), the relationship between the variation in pentane concentration, and the inlet entropy of the gas mixture is nearly a non-linear direct proportional relationship. As mentioned before, it can be concluded that the leaner feed generally give lower entropy content than those of rich feed. This is also like enthalpy due to the presence of high heavier hydrocarbons (C5+) components in the rich feed.

3.2.3. Effect of methane concentration variation on outlet entropy:

Fig.10 Effect of methane concentration variation on outlet entropy (kJ/kg.K)

From the illustrated figure (10), the relationship between the variation in methane concentration and the isentropic outlet entropy of the gas mixture is a close linear in an inverse proportion.

3.2.4. Effect of pentane concentration variation on the outlet entropy:

Fig.11: Effect of pentane concentration variation on the outlet entropy (kJ/kg.K)

As illustrated from the figure (11); the relationship between the variation in pentane concentration and the isentropic outlet entropy of the gas mixture is relatively a non-linear direct proportional relationship. As was mentioned before it can be concluded that the leaner feed generally give lower duty than those of rich feed. This is due to the presence of high heavier hydrocarbons (C5+) components in the rich feed.

3.3. Effect of feed composition on expander Efficiency:

3.3.1. Effect of methane concentration variation on expander Efficiency:

Fig.12: Effect of methane concentration variation on expander Efficiency (%)

As shown in the figure (12); the relationship between the variation in methane concentration and the expander efficiency of the gas mixture is a close linear in an inverse proportion. I.e., when the concentration of methane in the gas mixture increases, the expander efficiency decreases.

Since at standard condition the heat capacity at constant pressure (Cp) of methane is higher than other hydrocarbons present in the natural gas mixture, Cp of the mixture will be increased in a higher concentration of methane. When Cp of the fluid increases, the required compression work to reach a certain pressure will be decreased. Also, by increasing the concentration of methane in the mixture the molecular weight and the specific heat ratio (k=Cp/Cv) of the fluid will be decreased.

3.3.2 Effect of pentane concentration variation on expander Efficiency:

Fig.13: Effect of pentane concentration variation on expander Efficiency (%)

From the illustrated figure (13); the relationship between the variation in pentane concentration and expander efficiency of the gas mixture is a quasi-linear direct proportional relationship.

It is observed also the leaner feed generally give higher efficiency than those of rich feed. This is due to the presence of high heavier hydrocarbons (C5+) components in the rich feed.

3.4. Effect of feed composition on exergetic Efficiency:

3.4.1. Effect of methane concentration variation on exergetic Efficiency:

Fig.14: Effect of methane concentration variation on exergetic Efficiency (%)

As shown in the figure (14); the relationship between the variation in methane concentration and the exergetic efficiency of the gas mixture is a close linear in an inverse proportion. i.e., when the concentration of methane in the gas mixture increases, the exergetic efficiency decreases.

3.4.2 Effect of pentane concentration variation on exergetic Efficiency:

Fig.15: Effect of pentane concentration variation on exergetic Efficiency (%)

From the illustrated figure (15); the relationship between the variation in pentane concentration and the exergetic efficiency of the gas mixture is a quasi-linear direct proportional relationship. It is observed also the leaner feed generally give lower exergetic efficiency than those of rich feed. This is due to the presence of high heavier hydrocarbons (C5+) components in the rich feed.

The linearity deviation is attributed to inaccurate points. Mainly such points are of the rich samples, and for this reason, the exergetic efficiency is raised.

- Conclusions

Fifteen typical feed samples from ten gas reservoirs were obtained from four petroleum companies located at places that are spaced from each other between the Eastern & Western deserts of Egypt. Just six feeds from six gas reservoirs that have significant concentration differences were chosen as a study-base.

Math models were developed based on Peng-Robinson correlation to predict the performance data of a turbo-expander handling N.G. The models were solved by MATLAB software, validated and analyzed to get the most asymptotic model of reality to be further developed.

The obtained data are near those obtained by other researchers; 14 which mean that the model was successful in predicting the thermodynamic properties of different gas composition such as inlet & outlet enthalpies, inlet & outlet entropies, expander & exergetic efficiencies and isentropic outlet enthalpy. Validation was performed for verification of the results and shows good agreement with the calculated properties.

Nomenclature

| a | : | Measure of the Strength of Attraction between the Gas Molecules |
| --- | --- | --- |
| b | : | Volume Occupied by gas molecules, which decreases the available open volume |
| H | : | Enthalpy, kJ/kg |
| hi | : | Inlet Enthalpy, kJ/kg |
| ho | : | Outlet Enthalpy, kJ/kg |
| ho,s,e | : | Isentropic Outlet Enthalpy, kJ/kg |
| m .N.G | : | Natural Gas Mass Flow Rate |
| P | : | Pressure, barg |
| Pc | : | Critical Pressure, barg |
| Pi’ | : | Dimensionless Index in Novel Correlation |
| Ppc | : | Pseudo Critical Pressure for the mixture, barg |
| Pr | : | Reduced Pressure, dimensionless |
| PVP | : | Saturated Vapor Pressure of the gas at a temperature T = 0.7 Tc |
| Q | : | Inlet Volumetric Flowrate, m3/hr∙ |
| R | : | Universal Gas Constant, (Pa∙m3/mol∙k) |
| Si | : | Inlet Entropy, kJ/kg∙K |
| So,s,e | : | Isentropic Outlet Entropy, kJ/kg∙K |
| T | : | Temperature, K |
| Tc | : | Critical Temperature, K |
| Ti’ | : | Dimensionless Index in Novel Correlation |
| Tpc | : | Pseudo Critical Temperature for the mixture, K |
| Tr | : | Reduced Temperature, dimensionless |
| V | : | Gas Volume, m3 |
| Vm | : | Molar Volume, m3/mol |
| W .Gen | : | Electrical Power Obtained from the Exergy Recovery Process |
| W’Rev | : | Reversible Expansion Work |
| ϒi | : | Dimensionless Index in Novel Correlation |
| ϒi’ | : | Dimensionless Index in Novel Correlation |
| ω |  | acentric factor |

Reference

- KUMAR, MITHILESH. THERMODYNAMIC ANALYSIS OF AIR SEPARATION UNIT.2017.

- Popov, Dimityr, Kostadin Fikiin, Borislav Stankov, Graciela Alvarez, Mohammed Youbi-Idrissi, Alain Damas, Judith Evans, and Tim Brown. "Cryogenic heat exchangers for process cooling and renewable energy storage: A review." Applied Thermal Engineering 2019.

- Tafone, Alessio, Fabio Dal Magro, and Alessandro Romagnoli. "Integrating an oxygen enriched waste to energy plant with cryogenic engines and Air Separation Unit: Technical, economic and environmental analysis." Applied Energy 2018, 231, 423-432.

- Desideri, Adriano, Sergei Gusev, Martijn Van den Broek, Vincent Lemort, and Sylvain Quoilin. "Experimental comparison of organic fluids for low temperature ORC (organic Rankine cycle) systems for waste heat recovery applications." Energy 2016, 97, 460-469.

- Garcia, Steven Iglesias, Ramon Ferreiro Garcia, Jose Carbia Carril, and Denis Iglesias Garcia. "A review of thermodynamic cycles used in low temperature recovery systems over the last two years." Renewable and Sustainable Energy Reviews 2018, 81, 760-767.

- Feng, Yong-qiang, Tzu-Chen Hung, Ya-Ling He, Qian Wang, Shuang Wang, Bing-xi Li, Jaw-Ren Lin, and Wenping Zhang. "Operation characteristic and performance comparison of organic Rankine cycle (ORC) for low-grade waste heat using R245fa, R123 and their mixtures." Energy Conversion and Management 2017, 144, 153-163.

- Neseli, Mehmet Alparslan, Onder Ozgener, and Leyla Ozgener. "Energy and exergy analysis of electricity generation from natural gas pressure reducing stations." Energy Conversion and Management 2015, 93, 109-120.

- Olfati, Mohammad, Mehdi Bahiraei, Setareh Heidari, and Farzad Veysi. "A comprehensive analysis of energy and exergy characteristics for a natural gas city gate station considering seasonal variations." Energy 2018, 155, 721-733.

- Zhang, S., Wang, H., Li, R., Li, C., Hou, F., & Ben, Y. Thermodynamic analysis of cavern and throttle valve in large-scale compressed air energy storage system. Energy Conversion and Management 2019, 183, 721-731.

- Dai, B., Liu, S., Zhu, K., Sun, Z., & Ma, Y. Thermodynamic performance evaluation of transcritical carbon dioxide refrigeration cycle integrated with thermoelectric subcooler and expander. Energy 2017, 122, 787-800.

- Wronski, J., Imran, M., Skovrup, M. J., & Haglind, F. Experimental and numerical analysis of a reciprocating piston expander with variable valve timing for small-scale organic Rankine cycle power systems. Applied Energy 2019, 247, 403-416.

- Kim, Hyobin, Mungyu Kim, Youngrae Kim, Minseok Kim, Kihwan Lee, Minki Kim, Joonho Min, Donghun Lee, Jungmoon Jang, and Junghoon Kim. "Advanced Dual Refrigerant Expansion Cycle for Liquefaction." In Offshore Technology Conference. Offshore Technology Conference, 2018.

- Hossain, A., Pramanik, M. A. J., & Mullah, M. H. R. Review Of Thermodynamic Applications For NGL Extraction From Natural Gas In Cryogenic Processing. Journal of Nature Science and Sustainable Technology 2018, 12(2), 81-87.

- Jalali, A., Lotfi, M., Zilabi, S., & Mohammadi, A. H. Recovery enhancement of liquid hydrocarbons in dew point control unit of natural gas processing plant. Separation Science and Technology 2019, 1-8.

- Murali, Ashwin, Abdallah S. Berrouk, Satyadileep Dara, Yasser F. AlWahedi, Sulaimon Adegunju, Haitham Salmeen Abdulla, Anjan Kumar Das, Nafisa Yousif, and Mariam Al Hosani. "Efficiency enhancement of a commercial natural gas liquid recovery plant: A MINLP optimization analysis." Separation Science and Technology 2020, 55, no. 5, 955-966.

- Ahmadi, N., Rezazadeh, S., Dadvand, A., & Mirzaee, I.  Modelling of gas transport in proton exchange membrane fuel cells. Proceedings of the Institution of Civil Engineers-Energy 2016, 170(4), 163-179.

- GhasemiKafrudi, E., Amini, M., & Habibi, M. R.  Application of Turbo-Expander to Greenhouse Gas and Air Pollutant Emissions Reduction Using Exergy and Economical Analysis. Iranian Journal of Chemical Engineering 2017, 14(4), 33-47.

- Ghanaee, Reza, and Asghar Akbari Foroud. "Economic analysis and optimal capacity sizing of turbo-expander-based microgrid." IET Renewable Power Generation , 2016, 11, no. 4 , 511-520.

- Lopez-Echeverry, J. S., Reif-Acherman, S., & Araujo-Lopez, E.  Peng-Robinson equation of state: 40 years through cubics. Fluid Phase Equilibria 2017, 447, 39-71.

- Sodeifian, G., Ardestani, N. S., Sajadian, S. A., & Panah, H. S. Measurement, correlation and thermodynamic modeling of the solubility of Ketotifen fumarate (KTF) in supercritical carbon dioxide: evaluation of PCP-SAFT equation of state. Fluid Phase Equilibria 2018, 458, 102-114.

- Guo, G., Wang, F., Liu, G. Q., Luo, S. J., & Guo, R. B. Calculation on the phase equilibrium and critical temperature of CH4/CO2. Process Safety and Environmental Protection 2018, 113, 369-377.

- Sodeifian, G., Ardestani, N. S., & Sajadian, S. A. Solubility measurement of a pigment (Phthalocyanine green) in supercritical carbon dioxide: Experimental correlations and thermodynamic modeling. Fluid Phase Equilibria 2019, 494, 61-73.

- Farzaneh-Gord, M., & Rahbari, H.  Developing novel correlations for calculating natural gas thermodynamic properties. Chemical and Process Engineering 2011, 32(4), 435-452.

- Konukman, A. E. S., & Akman, U. Flexibility and operability analysis of a HEN-integrated natural gas expander plant. Chemical Engineering Science 2005, 60(24), 7057-7074.