Elemental Composition of Legumes Using ICP-MS and EDXRF Methodology Applications

Lovro Sinkovič,1, Marijan Nečemer,2 Barbara Pipan1 and Vladimir Meglič1

1 Crop Science Department, Agricultural Institute of Slovenia, Hacquetova ulica 17, SI-1000 Ljubljana, Slovenia

2 Department of Low and Medium Energy Physics, “Jožef Stefan” Institute, Jamova 39, SI-1000 Ljubljana, Slovenia

Abstract

In this research two analytical techniques, inductively coupled plasma-mass spectrometry (ICP-MS) and energy dispersive X-ray fluorescence (EDXRF) spectroscopy, were used to determine the elemental composition of different legumes usually produced and consumed in Slovenia. Results indicate that data obtained using these methods are in good agreement with certified reference materials. In total, nineteen elements were determined in twenty legume samples. An intercomparing for four macro- (P, S, K, Ca) and three microelements (Fe, Zn, Mo) measured using ICP-MS and EDXRF methods showed a strong correlation. The EDXRF was found cheaper, a simpler and more environmentally friendly method for analysis of elements P, S, Cl, K, Ca, Fe, Zn, Mo, Sr, Rb, Ti and Br in legumes, while for the identification and determination of Na, Mg, V, Cr, Mn, Co and Cu content ICP‐MS was a method of choice due to its excellent sensitivity and accuracy. By principal component analysis (PCA), the samples of the studied legumes were classified into four groups according to their elemental composition.

Keywords: EDXRF; elemental composition; ICP-MS; legumes.

Povzetek

V raziskavi smo z dvema analitskima tehnikama, induktivno sklopljeno plazemsko-masno spektrometrijo (ICP-MS) in energijsko disperzijsko rentgensko fluorescenčno spektroskopijo (EDXRF), določili elementarno sestavo različnih stročnic, ki se pogosto pridelujejo in uživajo v Sloveniji. Rezultati kažejo, da se podatki, pridobljeni z uporabo teh metod, dobro ujemajo s certificiranimi referenčnimi materiali. Skupno smo v dvajsetih vzorcih stročnic določili devetnajst elementov. Medsebojna primerjava štirih makro- (P, S, K, Ca) in treh mikroelementov (Fe, Zn, Mo) izmerjenih z ICP-MS in EDXRF, je pokazala visoko korelacijo med uporabljenima metodama. EDXRF se je izkazala za cenejšo, enostavnejšo in okolju prijaznejšo metodo za določanje elementov P, S, Cl, K, Ca, Fe, Zn, Mo, Sr, Rb, Ti in Br v stročnicah, medtem ko je za določanje vsebnosti Na, Mg, V, Cr, Mn, Co in Cu ustreznejša ICP-MS metoda, predvsem zaradi visoke občutljivosti in natančnosti. Z analizo glavnih komponent (PCA) smo uspeli razvrstiti vzorce preučevanih stročnic glede na elementarno sestavo v štiri skupine.

- Introduction

Legumes are of prime importance in human or animal nutrition and contain a great variety of plants.1 These include crops grown for grains (e.g. common bean, runner bean, lupins, lentil, chickpea), fresh vegetables (e.g. snap bean, green pea) and livestock forage (e.g. soybean, field pea). Legumes play a distinct role in agricultural ecosystems with their ability to fix nitrogen symbiotically.2 Grains of food legumes are an important source of elements such as P, Ca, K, N, Fe, Mg and Zn, essential for a human being. Legume grains not only play a vital role in many traditional diets worldwide but are valuable for the food and animal feed industries.3 The common bean is the most important grain legume for direct human consumption which provides 10 – 20% of the adult requirements for nutrients, namely Fe, P, Mg, Mn, and to a lesser degree for Zn, Cu and Ca.4 Chickpea applies the third most important grain legume for human consumption after bean and pea.5 According to the FAO, in 2019 the total world production was the highest for soybeans, followed by beans (Phaseolus spp.), peas, chickpeas, lentils, faba beans and lupins.6

The main objective in multi‐elemental analysis of foods is to ensure food quality and safety. Therefore, with the increased crop production to meet the growing demands, product quality becomes an important issue.7 Elemental fingerprinting has been proven effective way for the quality and authentication of foods.8 Besides, it serves as an important tool for plant breeding programmes and nutritional biofortification purposes.9,10,11 Around twenty of the known elements are defined as essential since they act as important phytochemicals and have a significant role in the maintenance of human health. Essential elements are classified into macroelements or major minerals, namely Na, K, Mg, Ca, Cl, P and S, and microelements or trace minerals such as Fe, Cu, Mn, Zn, Co and Mo.12 The levels of these elements are measured to provide valuable nutritional information about foods. Over the past decade, with the development of advanced analytical techniques, the element composition of various food samples can be successfully measured using inductively coupled plasma-mass spectrometry (ICP-MS), energy dispersive X-ray fluorescence (EDXRF) spectroscopy, etc.13 Although ICP-MS is frequently the most accurate technique for elemental analysis because of the simple, easily interpreted spectra and the exceptionally low limits of detection, on the other hand, requires expensive reagents, gases and laborious sample preparation.8 EDXRF spectroscopy is a good alternative to ICP-based methods, in which sample digestions are needed since hardly any sample treatment is required to carry out multi-elemental analysis in solid samples. Despite huge progress that has been made in EDXRF instruments fewer works were published in the field of elemental characterization of organic matrices such as plant foods by this technique. Nevertheless, EDXRF is characterized by limits of detection at the low mg/kg level, which are about three orders of magnitude higher than those obtained by the ICP-MS method.14

Within the framework of the current studies at the Agricultural Institute of Slovenia, the capabilities of two analytical techniques, ICP-MS and EDXRF spectroscopy, were compared by the analysis of several legume samples. The present research tried to provide data on elemental composition for legumes usually produced and consumed in Slovenia which can be used in the ongoing plant breeding programs (e.g. common bean). The purpose of the study was: (i) to determine the multi-elemental composition of different legumes; (ii) to assess the appropriateness of two analytical methods ICP-MS and EDXRF for element determination of different legumes; and (iii) to evaluate the correlation between used analytical methods.

- Experimental

  - Materials

A set of twenty homogenised plant samples consists of several legume species as presented in Table 1. Analysed samples were as follows: common bean grains (6 samples), common bean pods (3 samples), lupin (3 samples), faba bean (2 samples), lentil (2 samples), chickpea (1 sample), soybean (1 sample), field pea (1 sample) and runner bean (1 sample). Most of the analysed legumes were produced at the experimental fields of Infrastructure Centre Jablje at the Agricultural Institute of Slovenia (304 m a.s.l.; 46.151°N 14.562°E). Chickpea, brown and red lentil samples were purchased from the Slovenian food retail market. The legume grain samples were air-dried after harvest to reduce the moisture content below 11%. The common bean pods were immediately after harvesting frozen using liquid nitrogen and lyophilized. Before elemental analysis, all legume samples were homogenised and powdered using a laboratory ball mill (Retsch MM 400, GmbH) at a high frequency of 30 Hz for 2–5 min.

Table 1. List of studied legume samples and their origin

| Sample name | Legume species | Latin name | Variety/ accession | Seed provider | Sample type |
| --- | --- | --- | --- | --- | --- |
| KIS_GL1 | common bean | Phaseolus vulgaris L. | Ribnčan | Semenarna Ljubljana | grains |
| KIS_GL2 | common bean | Phaseolus vulgaris L. | SRGB204 | Slovenian Plant Gene Bank | grains |
| KIS_GL3 | common bean | Phaseolus vulgaris L. | Zorin | Semenarna Ljubljana | grains |
| KIS_GL4 | common bean | Phaseolus vulgaris L. | Ribnčan | Semenarna Ljubljana | pods |
| KIS_GL5 | common bean | Phaseolus vulgaris L. | SRGB204 | Slovenian Plant Gene Bank | pods |
| KIS_GL6 | common bean | Phaseolus vulgaris L. | Zorin | Semenarna Ljubljana | pods |
| KIS_GL7 | common bean | Phaseolus vulgaris L. | Etna | Semenarna Ljubljana | grains |
| KIS_GL8 | common bean | Phaseolus vulgaris L. | Golden gate | Semenarna Ljubljana | grains |
| KIS_GL9 | common bean | Phaseolus vulgaris L. | SRGB196 | Slovenian Plant Gene Bank | grains |
| KIS_GL10 | white lupin | Lupinus albus L. | Energy | Feldsaaten Freudenberger | grains |
| KIS_GL11 | faba bean | Vicia faba L. var. minor | Zoran | Agricultural Institute of Slovenia | grains |
| KIS_GL12 | faba bean | Vicia faba L. var. minor | Merkur | Semenarna Ljubljana | grains |
| KIS_GL13 | blue lupin | Lupinus angustifolius L. | Sonet | Feldsaaten Freudenberger | grains |
| KIS_GL14 | yellow lupin | Lupinus luteus L. | Mister | Feldsaaten Freudenberger | grains |
| KIS_GL15 | red lentil | Lens culinaris Medik. | rdeča leča | food retail market | grains |
| KIS_GL16 | brown lentil | Lens culinaris Medik. | rjava leča | food retail market | grains |
| KIS_GL17 | chickpea | Cirer arietinum L. | čičerika | food retail market | grains |
| KIS_GL18 | soybean | Glycine max L. Merr. | ES Mentor | Saatbau Slovenia | grains |
| KIS_GL19 | field pea | Pissum sativum L. | Eso | Semenarna Ljubljana | grains |
| KIS_GL20 | runner bean | Phaseolus coccineus L. | SRGB222 | Slovenian Plant Gene Bank | grains |

  - Multi-elemental analysis using ICP-MS

Digestion of plant samples (decomposition of organic matter). The powdered legume samples were digested using a high-pressure microwave oven (Milestone ETHOS 1600). Separate samples were weighed (250 mg) into PTFE vessels and 6 mL of 65% nitric acid (HNO3, SUPRAPUR, Merck) and 2 mL 30% hydrogen peroxide (H2O2, SUPRAPUR, Merck) was added. The digestion was conducted according to the following programme: step 1, 300 W, 5 min at 100 °C; step 2, 400 W, 5 min at 150 °C; step 3, 500 W, 5 min at 180 °C; step 4, 600 W, 7 min at 210 °C; step 5, 550 W, 15 min at 210 °C; step 6, 0 W, 20 min cooling.

Preparation of test solution. Digested samples were cooled to room temperature and the solution was quantitatively transferred into 50 mL plastic tubes (Sarstedt, USA) and filled up to full volume with Milli-Q water. Before analysis, the digested samples were diluted by a factor of 20 and consisted of 1% HNO3 (v/v).

Determination of elements by ICP-MS. For the determination of elements in samples, the Agilent 7900 ICP-MS was used. This instrument includes a 4th generation collision/reaction cell, the Octopole reaction System (ORS4) which provides optimized operational conditions for the removal of polyatomic interferences using helium (He) collision mode. In this manner, smaller and faster analyte ions are separated from larger, slower interference ions using kinetic energy discrimination. The following isotopes were monitored: 23Na, 24Mg, 31P, 34S, 39K, 43Ca, 51V, 52Cr, 55Mn, 56Fe, 59Co, 63Cu, 66Zn and 95Mo. Due to the high sensitivity of the 7900 ICP-MS most elements of interest could be measured in He mode, only phosphorus (P) and sulphur (S) were measured in high helium (HEHe) mode. The ICP-MS operating conditions were optimized using autotuning functions within the ICP-MS MassHunter software. Other instruments operating conditions were as follows: general-purpose plasma mode; peri-pump sample introduction; micro-mist nebuliser; nickel cones interface; He gas flow was 5 mL/min in He mode and 10 mL/min in HEHe mode; spectrum acquisition mode; one point peak pattern; three replicates and 100 sweeps per replicate.

Calibration of the instrument. Quantitative analysis was performed with the external calibration method. Calibration standards for most of the elements were prepared using IV-STOCK-50 standard solution containing: 1000 mg/L of Na, Mg, K, Ca and Fe and 10 mg/L of V, Cr, Mn, Co, Cu, Zn and Mo (matrix 5% v/v HNO3, Inorganic Ventures). Phosphorus (1000 mg/L P, matrix H2O, CGP1, Inorganic Ventures) and sulphur (1000 mg/L S, matrix H2O, CGS1, Inorganic Ventures) single standard solutions were added separately to the mixture. A five-point calibration from 0.05 µg/L to 50 µg/L was carried out for the elements V, Cr, Mn, Co, Cu, Zn and Mo, and from 5 µg/L to 5000 µg/L for the elements Na, Mg, S, P, K and Fe. Only for Ca a six-point calibration was used between 5 µg/L and 10000 µg/L due to the higher concentrations expected in the plant samples. Final multi-element calibration solutions were prepared daily and contained 1% HNO3 (v/v). The internal standards used to compensate for sensitivity drift and matrix effects during the analytical run were Sc, Rh, In and Lu (Agilent PN 5188-6525). They were added online in a concentration of 200 µg/L (in 1% v/v HNO3).

Quality control. For quality control analytical blanks, independent QC standards and standard reference material (SRM) were used. In each test series, a blank sample containing only acids was included. The QC standards were prepared in the concentrations of 2 µg/L, 20 µg/L and 2000 µg/L over the analytical range from ICP-MS multi-standard solution VIII (MERCK, Certipur, PN 1.09492) and ICP-MS multi-standard solution XVI (MERCK, Certipur, PN 1.09487) and were analysed in the beginning, in the middle and at the end of each analysis run. Finally, a NIST SRM Tomato Leaves (1573a) was used to check the accuracy of the analytical procedures and recovery. Analytical data were processed using the ICP-MS MassHunter Workstation Software (Rev. C.01.02, G7201C, Agilent technologies, 2015). The software calculates the correlation coefficient of the calibration curve (R), the limit of detection (LOD) and the background equivalent concentration (BEC) for each element. These data along with the accuracy (as % recovery) are shown in Table 2. The data are expressed as macro- (g/kg) or microelements (mg/kg).

Table 2. ICP-MS calibration coefficients, method detection limits and background equivalent concentration data

| Element | R2 | LOD (mg/kg) | BEC (mg/kg) | Recovery (%) |
| --- | --- | --- | --- | --- |
| 23Na | 1.0000 | 1.07 | 6.63 | 82.5 |
| 24Mg | 1.0000 | 0.53 | 0.74 | 84.9 |
| 31P | 1.0000 | 10.8 | 8.2 | 92.7 |
| 34S | 1.0000 | 223 | 3360 | 95.7 |
| 39K | 1.0000 | 6.8 | 318 | 91.4 |
| 43Ca | 1.0000 | 48 | 125 | 89.9 |
| 51V | 1.0000 | 0.0001 | 0.0041 | 71.8 |
| 52Cr | 1.0000 | 0.085 | 0.342 | 86.6 |
| 55Mn | 1.0000 | 0.038 | 0.093 | 90.9 |
| 56Fe | 1.0000 | 0.021 | 3.522 | 86.3 |
| 59Co | 1.0000 | 0.003 | 0.035 | 85.8 |
| 63Cu | 1.0000 | 0.041 | 0.16 | 98.5 |
| 66Zn | 1.0000 | 0.254 | 0.759 | 89.7 |
| 95Mo | 1.0000 | 0.01 | 0.217 | 73.3 |

R, calibration coefficient; LOD, method detection limit; BEC, background equivalent concentration.

  - Multi-elemental analysis using EDXRF

Identification of twelve elements (P, S, Cl, K, Ca, Ti, Fe, Zn, Br, Rb, Sr, Mo) in a single measurement was carried out using non-destructive EDXRF spectrometry. Pellets made from 0.5 g to 1.0 g of powdered legume samples were prepared using a pellet die and a hydraulic press. The disc radioisotope excitation sources Fe-55 (25 mCi) and Cd-109 (20 mCi) from Eckert & Ziegler (Germany) were used for fluorescence excitation. The emitted fluorescence radiation was measured using the EDXRF spectrometer with an XR-100 SDD detector (Amptek), a PX5 digital pulse processor (Amptek), and a PC-based, multichannel analyser software package DPPMCA. In Fe-55 mode, the spectrometer was equipped with a vacuum chamber to measure the three light elements (P, S, Cl), and in Cd-109 mode, the multi-element measurement was performed in the air for the nine elements K, Ca, Ti, Fe, Zn, Br, Rb, Sr and Mo. The energy resolution of the spectrometer was 125eV at 5.9keV. The analysis of complex X-ray spectra was performed using the AXIL spectral analysis program.15 The evaluated uncertainty of this procedure included the statistical uncertainty of measured intensities and the uncertainty of the mathematical fitting procedure. The overall uncertainty of spectral measurement and analysis was in most cases better than 1%.

Quantification was performed using the QAES (Quantitative Analysis of Environmental Samples) software.16 The estimated uncertainty of the analysis was around 5% to 10%, LOD for Zn was from 5 μg/g to 10 μg/g. Rather a high total estimated uncertainty is mainly due to contributions of matrix correction and geometry calibration procedures, which include errors of tabulated fundamental parameters, and also contributions of spectrum acquisition and analysis. The uncertainty due to the inhomogeneity of the sample was not included. The quantified twelve elements were expressed as macro- (g/kg) or microelements (mg/kg).

  - Statistical evaluation

Statistical calculations and multivariate analysis were carried out using the XLSTAT software package (Addinsoft, New York, USA). The multivariate analysis involved principal component analysis (PCA).

- Results and Discussion

A common characteristic of both analytical techniques applied ICP-MS and EDXRF is their multi-element capability. Preparation of legume samples was simple and non-destructive in the case of EDXRF, meanwhile, ICP-MS required skilled personnel and decomposition of samples. ICP-MS was a more sensitive method in this study with LODs in the range of ng/g. Results of the LOD, BEC and accuracy of multi-elemental analysis performed by ICP-MS with certified reference material (NIST SRM Tomato Leaves 1573a) are presented in Table 1. The sensitivity of EDXRF according to estimated uncertainty was from 5% to 10% and LODs for the analysed elements in the range from a hundred to a few μg/g. This means that LODs of ICP-MS were approx. two orders of magnitude levered compared to EDXRF. The determination of element Cl by ICP-MS was impossible since form negative ions while EDXRF enables its analysis as an essential element (dietary mineral) and one of the main electrolytes in the body. On the other hand, ICP-MS enables the determination of elements Na, Mg, Mn and Cu which are according to the European Food Safety Authority (EFSA) essential substances required as nutrients by the body to perform a variety of functions. Considering the cost per sample, EDXRF was cheaper, simpler and more environmentally friendly compared to ICP-MS and more suitable for analysis of P, S, Cl, K, Ca, Fe, Zn, Mo, Sr, Rb, Ti and Br in legume samples. However, for analysis of Na, Mg, V, Cr, Mn, Co and Cu content ICP‐MS was a method of choice due to its excellent sensitivity and accuracy.

The macroelement composition of twenty legume samples determined by ICP-MS and/or EDXRF is presented in Table 3 and the microelement composition in Table 4. A total of nineteen elements were determined and divided into six macro- (Mg, P, S, K, Cl, Ca) and thirteen microelements (Mn, Fe, Cu, Na, Cr, Co, Zn, V, Rb, Ti, Br, Sr, Mo). The results of macroelements are depicted as g/kg (Table 3) and those of microelements as mg/kg (Table 4). Based on the average values the order is K > P > S > Ca > Mg > Cl of for the macro- and Fe > Mn > Zn > Na > Cu > Rb > Br > Ti > Sr > Mo > Cr > Co > V for the microelements in analysed legume samples. The ranks of individual macroelements in the analysed legumes were as follows: K (7.5–22.6 g/kg), P (2.7–7.5 g/kg), S (0.9–4.7 g/kg), Ca (0.2–5.0 g/kg), Mg (0.8–2.9 g/kg), and Cl (0.1–51.0g/kg). Here, the highest coefficient of variation was calculated for the Ca (66.49%), followed by Cl (58.30%) and S (38.40%). Among different legume species, the highest content of K was found for common bean pods (snap beans) and soybean grains and the lowest for lentil. The P content was the highest for faba bean and the lowest for chickpea. The lupins and soybean contained the most S, and common bean pods the most Ca. Compared to other legumes, lentil had the lowest content of Ca and Mg. The ranks of essential microelements in the analysed legumes were the following: Fe (37.2–126.0 mg/kg), Mn (8.3–487.7 mg/kg), Zn (17.5–64.6 mg/kg), Na (1.7–91.8 mg/kg), Cu (4.9–16.3 mg/kg), and Mo (0.2–8.5 mg/kg). Among determined microelements the highest coefficients of variation were calculated for the Mn, Cr and Na (> 102.12%) while the lowest for Cu, Zn and Fe (< 38.72%). The highest content of Fe was found for soybean and the lowest for field pea. All three lupin samples (white, blue and yellow) was the richest source of microelement Mn compared to other legumes. The Na content was the highest for common bean pods and lupins. These elemental compositions are consistent with literature data for common bean grains17,18 and pods,19,20 lupin,21,22 faba bean,22,23 lentil,3,24,25 chickpea,3,5 soybean,21 field pea25 and runner bean.26,27

Table 3. Macroelement composition of 20 legume samples determined by ICP-MS and EDXRF

| Sample name | Legume species | Macroelements (g/kg) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | K |  | P |  | S |  | Ca |  | Mg | Cl |
|  |  | ICP-MS | ED XRF | ICP-MS | ED XRF | ICP-MS | ED XRF | ICP-MS | ED XRF | ICP-MS | ED XRF |
| KIS_GL1 | CB (grains) | 13.9 | 12.8 | 4.8 | 5.0 | 2.1 | 1.9 | 1.0 | 0.8 | 1.4 | 0.2 |
| KIS_GL2 | CB (grains) | 19.1 | 17.3 | 5.9 | 5.9 | 2.3 | 1.9 | 1.2 | 1.0 | 1.6 | 0.3 |
| KIS_GL3 | CB (grains) | 14.4 | 13.1 | 5.0 | 4.6 | 2.2 | 1.4 | 0.9 | 0.9 | 1.5 | 0.1 |
| KIS_GL4 | CB (pods) | 17.1 | 18.3 | 3.1 | 3.5 | 1.4 | 1.2 | 2.5 | 2.2 | 1.7 | 0.7 |
| KIS_GL5 | CB (pods) | 18.5 | 18.7 | 3.1 | 3.9 | 1.7 | 1.5 | 4.1 | 3.3 | 2.0 | 0.6 |
| KIS_GL6 | CB (pods) | 20.9 | 22.6 | 3.7 | 4.7 | 1.5 | 1.8 | 5.0 | 3.7 | 2.1 | 1.0 |
| KIS_GL7 | CB (grains) | 13.5 | 13.4 | 4.2 | 4.5 | 2.0 | 1.5 | 1.3 | 1.1 | 1.5 | 0.1 |
| KIS_GL8 | CB (grains) | 16.3 | 15.4 | 5.9 | 5.8 | 2.4 | 2.1 | 1.2 | 1.1 | 1.7 | 0.3 |
| KIS_GL9 | CB (grains) | 17.8 | 15.9 | 5.9 | 5.6 | 2.2 | 1.7 | 0.8 | 0.9 | 1.7 | 0.1 |
| KIS_GL10 | white lupin | 15.3 | 11.9 | 5.3 | 6.2 | 2.4 | 2.5 | 3.0 | 1.8 | 1.7 | 0.1 |
| KIS_GL11 | faba bean | 12.5 | 11.4 | 6.5 | 6.5 | 1.5 | 0.9 | 1.1 | 0.9 | 1.4 | 0.6 |
| KIS_GL12 | faba bean | 12.7 | 12.9 | 6.2 | 6.5 | 1.6 | 1.1 | 1.0 | 1.0 | 1.4 | 0.5 |
| KIS_GL13 | blue lupin | 11.1 | 10.4 | 4.9 | 4.9 | 2.2 | 2.0 | 2.2 | 2.1 | 1.8 | 0.2 |
| KIS_GL14 | yellow lupin | 12.1 | 11.6 | 7.5 | 7.0 | 4.7 | 4.6 | 1.9 | 1.6 | 2.9 | 0.6 |
| KIS_GL15 | red lentil | 9.2 | 9.1 | 3.7 | 3.5 | 1.8 | 1.5 | 0.2 | 0.2 | 0.8 | 0.5 |
| KIS_GL16 | brown lentil | 7.8 | 7.5 | 3.1 | 3.1 | 1.6 | 1.2 | 0.8 | 0.8 | 0.8 | 0.7 |
| KIS_GL17 | chickpea | 10.1 | 9.4 | 2.9 | 2.7 | 2.1 | 1.4 | 1.3 | 1.1 | 1.3 | 0.6 |
| KIS_GL18 | soybean | 18.2 | 17.1 | 5.5 | 4.9 | 3.0 | 2.5 | 1.8 | 1.6 | 2.4 | 0.1 |
| KIS_GL19 | field pea | 9.1 | 8.8 | 3.7 | 3.6 | 1.6 | 1.3 | 0.9 | 0.8 | 1.3 | 0.5 |
| KIS_GL20 | runner bean | 17.8 | 16.3 | 4.6 | 5.1 | 2.0 | 2.3 | 1.1 | 1.2 | 1.8 | 0.5 |
| Correlation |  | 0.96 |  | 0.94 |  | 0.94 |  | 0.98 |  | n.a. | n.a. |
| Coefficient of variation (%) |  | 26.85 |  | 25.54 |  | 38.40 |  | 66.49 |  | 29.27 | 58.30 |

CB, common bean; n.a., not applicable.

Table 4. Microelement composition of 20 legume samples determined by ICP-MS and EDXRF

| Sample name | Legume species | Microelements (mg/kg) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Fe |  | Mn | Zn |  | Na | Cu | Rb | Br | Ti | Sr | Mo |  | Cr | Co | V |
|  |  | ICP-MS | ED XRF | ICP-MS | ICP-MS | ED XRF | ICP-MS | ICP-MS | ED XRF | ED XRF | ED XRF | ED XRF | ICP-MS | ED XRF | ICP-MS | ICP-MS | ICP-MS |
| KIS_GL1 | CB (grains) | 64.6 | 69.2 | 12.0 | 28.1 | 26.6 | 6.1 | 8.7 | 5.9 | 1.3 | 1.6 | 0.9 | 0.6 | 0.9 | 0.2 | 0.03 | 0.003 |
| KIS_GL2 | CB (grains) | 69.9 | 79.0 | 12.4 | 27.9 | 26.7 | 3.3 | 9.5 | 3.7 | 1.2 | 2.2 | 0.9 | 3.2 | 1.9 | 0.1 | 0.04 | 0.000 |
| KIS_GL3 | CB (grains) | 57.7 | 91.6 | 11.0 | 28.9 | 22.3 | 2.4 | 5.9 | 9.0 | 1.7 | 3.5 | 1.1 | 0.9 | 0.9 | 0.3 | 0.03 | 0.004 |
| KIS_GL4 | CB (pods) | 54.0 | 65.8 | 13.6 | 21.5 | 17.5 | 67.2 | 7.1 | 6.2 | 6.9 | 3.5 | 3.0 | 0.4 | 1.1 | 0.1 | 0.03 | 0.026 |
| KIS_GL5 | CB (pods) | 44.2 | 76.6 | 14.7 | 22.0 | 23.9 | 53.5 | 5.8 | 3.9 | 2.9 | 4.4 | 4.1 | 0.2 | 1.3 | 0.1 | 0.02 | 0.002 |
| KIS_GL6 | CB (pods) | 50.6 | 68.0 | 16.7 | 24.2 | 25.4 | 91.8 | 4.9 | 8.2 | 13.9 | 3.3 | 4.8 | 0.4 | 1.8 | 0.2 | 0.02 | 0.014 |
| KIS_GL7 | CB (grains) | 77.3 | 84.0 | 13.6 | 26.8 | 25.1 | 3.0 | 8.3 | 2.4 | 1.5 | 3.0 | 4.6 | 5.8 | 4.5 | 0.2 | 0.04 | 0.045 |
| KIS_GL8 | CB (grains) | 58.7 | 76.0 | 11.9 | 27.6 | 27.1 | 38.3 | 5.8 | 7.8 | 1.8 | 2.4 | 3.1 | 8.5 | 6.6 | 0.2 | 0.15 | 0.019 |
| KIS_GL9 | CB (grains) | 56.9 | 84.7 | 9.4 | 20.3 | 19.4 | 1.7 | 5.2 | 6.7 | 1.6 | 3.8 | 0.8 | 4.7 | 3.5 | 0.2 | 0.05 | 0.008 |
| KIS_GL10 | white lupin | 37.2 | 57.5 | 487.7 | 42.4 | 48.6 | 58.8 | 8.0 | 23.3 | 2.0 | 2.0 | 2.0 | 2.2 | 1.6 | 0.1 | 0.09 | 0.008 |
| KIS_GL11 | faba bean | 43.3 | 74.7 | 16.4 | 49.1 | 43.1 | 9.0 | 16.3 | 5.6 | 1.3 | 2.0 | 0.9 | 0.9 | 1.1 | 0.2 | 0.20 | 0.005 |
| KIS_GL12 | faba bean | 46.9 | 84.9 | 16.8 | 44.6 | 43.0 | 10.0 | 16.3 | 3.4 | 1.5 | 2.1 | 1.8 | 1.0 | 0.9 | 0.2 | 0.16 | 0.007 |
| KIS_GL13 | blue lupin | 43.9 | 58.3 | 79.5 | 34.9 | 33.1 | 58.6 | 6.1 | 8.9 | 3.5 | 2.4 | 5.7 | 1.7 | 1.4 | 1.7 | 0.04 | 0.031 |
| KIS_GL14 | yellow lupin | 64.9 | 95.6 | 82.9 | 63.3 | 64.6 | 78.3 | 10.7 | 3.3 | 2.1 | 4.1 | 2.2 | 1.4 | 1.1 | 2.9 | 0.07 | 0.022 |
| KIS_GL15 | red lentil | 65.9 | 87.6 | 13.6 | 33.0 | 33.0 | 10.7 | 9.1 | 4.1 | 3.8 | 2.7 | 1.8 | 5.3 | 4.4 | 0.1 | 0.03 | 0.008 |
| KIS_GL16 | brown lentil | 47.3 | 70.8 | 9.6 | 19.6 | 22.1 | 6.9 | 8.1 | 3.1 | 1.3 | 2.8 | 1.8 | 0.5 | 0.9 | 0.2 | 0.04 | 0.006 |
| KIS_GL17 | chickpea | 55.5 | 71.1 | 25.8 | 28.6 | 26.0 | 33.8 | 8.0 | 6.7 | 10.8 | 0.6 | 5.5 | 2.7 | 1.7 | 1.0 | 0.12 | 0.027 |
| KIS_GL18 | soybean | 86.0 | 126.0 | 22.7 | 36.8 | 35.9 | 10.3 | 13.4 | 7.6 | 6.6 | 3.7 | 2.0 | 0.7 | 2.0 | 2.7 | 0.13 | 0.034 |
| KIS_GL19 | field pea | 46.8 | 79.3 | 8.3 | 26.3 | 24.3 | 16.2 | 7.3 | 4.9 | 1.6 | 1.1 | 1.8 | 0.6 | 1.0 | 0.2 | 0.03 | 0.027 |
| KIS_GL20 | runner bean | 52.7 | 77.6 | 12.6 | 23.0 | 20.8 | 1.8 | 5.4 | 3.4 | 2.6 | 1.0 | 2.6 | 2.4 | 1.9 | 0.3 | 0.04 | 0.013 |
| Correlation |  | 0.72 |  | n.a. | 0.97 |  | n.a. | n.a. | n.a. | n.a. | n.a. | n.a. | 0.95 |  | n.a. | n.a. | n.a. |
| Coefficient of variation (%) |  | 25.96 |  | 232.72 | 35.83 |  | 102.12 | 38.72 | 68.38 | 97.13 | 39.79 | 59.72 | 89.18 |  | 146.10 | 77.30 | 79.13 |

CB, common bean; n.a., not applicable.

In nutrition, minerals are inorganic substances required as nutrients by the body to perform a variety of functions. The EFSA set dietary reference values for the following fourteen essential elements Ca, Cl, Cu, F, I, Fe, Mg, Mn, Mo, P, K, Se, Na and Zn. Using in-house developed ICP-MS multi-element method for plant samples ten (Ca, Cu, Fe, Mg, Mn, Mo, P, K, Na, Zn) of these fourteen elements were determined. The three elements, namely Cl, F and I, belong to halogen elements and for these due to the presence of interferences formed in the Ar plasma operating under conventional conditions or to matrix effects, their determination by ICP-MS is still considered a challenging task. Despite quadrupole ICP-MS is by far the most used instrumentation for multi-element analysis, there are some drawbacks particularly associated with its application for halogen determination.28 Furthermore, the Se is another element for which the determination by ICP-MS is a challenge. For Se analyses, however, conventional quadrupole ICP-MS operation suffers from inadequate sensitivity due to the high ionization potential of Se in the plasma as well as isobaric and polyatomic interferences. Selenium has six stable isotopes (74Se 0.87%, 76Se 9.02%, 77Se 0.58%, 78Se 23.52%, 80Se 49.82%, 82Se 9.19%), which adds complexity to the analysis.29 When using the EDXRF multi-element method seven (Ca, Cl, Fe, Mo, P, K, Zn) of fourteen above mentioned elements were determined. However, EDXRF was a non-destructive and simultaneous method with simple sample preparation steps.30 After the sample preparation procedure, the EDXRF analysis presented high analytical frequency and most equipment counts on an automatic sample holder.31

Out of a total of nineteen elements determined using ICP-MS and EDXRF, the following seven P, S, K, Ca, Fe, Zn and Mo were determined by both methods. The correlations between these two methods for the macroelements P, S, K and Ca are presented in Table 3 and for the microelements Fe, Zn and Mo in Table 4. A very strong correlations were detected for the elements Ca (0.98), Zn (0.97), K (0.96), Mo (0.95), P and S (0.94), while strong correlation was observed for Fe (0.72). For the elements K, S, Ca and Zn there was a tendency that EDXRF values were lower compared to ICP-MS values. Phosphorus values were comparable between methods while Fe EDXRF values were higher compared to ICP-MS values. These discrepancies can be due to the inhomogeneity of the samples.

Statistical evaluation of results was performed on the dataset by PCA to identify the elements responsible for differentiating legume samples. The discrimination score plot and loadings across the original dataset are shown in Fig. 1. When the PCA was applied to the data (20 samples, 19 variables), discriminant functions were obtained where Function 1 explained 22.31% and Function 2 21.90% of the total variance. According to the multi-elemental composition, the analysed legumes can be divided into four groups as seen in Fig. 1. The first group included samples of common bean pods and chickpea with the most influential elements Ca, Na, Sr and Cl. Second group involved soybean and lupin samples where the most influential elements were Na, Mg, S, Zn and P. The third group consisted of runner bean, field pea and lentil samples with most influential elements Mg, S, Zn and P. Finally, the faba bean and common bean grain samples formed the fourth group with most influential elements Mo, Cu and Co.

Fig. 1. PCA score plot (left) and discriminant loadings plot (right)

- Conclusion

There is a need for reliable legumes element concentration data to provide information about nutritional uptake, especially for essential macro- and microelements. Besides, the multi-elemental composition provides important information for the plant breeding programmes and can potentially serve as a base for nutrient biofortification purposes (e.g. Fe, Zn). This paper provides some interesting comparisons between two different techniques (ICP-MS, EDXRF) in the determination of the multi-elemental composition of different legumes. An intercomparing for seven elements (P, S, K, Ca, Fe, Zn and Mo) showed satisfactory agreement between both methods. The simple, fast and cheaper EDXRF method in combination with ICP-MS, which was found the appropriate technique for analysis of Na, Mg, V, Cr, Mn, Co and Cu, were used to provide the first evaluation of the use of multi-elemental composition to differentiate between several legume samples usually produced and consumed in Slovenia. Despite these encouraging data, there remain some limitations and further research base on a larger dataset of each legume species will be important in establishing more reliable databases in elemental composition.

- Acknowledgements

This study was a part of the research programme Agrobiodiversity (P4-0072) and the research project L4-7520 financially supported by the Slovenian research agency (ARRS), Ljubljana, Slovenia, and the Public Service in vegetables and herbs financed by the Ministry of Agriculture, Forestry and Food, Republic of Slovenia. The authors would like to thank Peter Kump, PhD, for kindly providing the QAES (Quantitative Analysis of Environmental Samples) software (XRF), and to the national curator of SPGB (Slovene Plant Gene Bank) Jelka Šuštar Vozlič, PhD, for providing the legumes genetic resources.

- Reference

- E. Röös, G. Carlsson, F. Ferawati, M. Hefni, A. Stephan, P. Tidåker, C. Witthöft, Renew. Agr. Food Syst. 2020, 35(2), 192–205. DOI: 10.1017/S1742170518000443

- D. Rubiales, A. Mikić, Crit. Rev. Plant Sci., 2015, 34, 2–3. DOI: 10.1080/07352689.2014.897896

- M. Farooq, M. Hussain, M. Usman, S. Farooq, S. S. Alghamdi, K. H. Siddique, J. Agric. Food Chem. 2018, 66(34), 8887–8897. DOI: 10.1021/acs.jafc.8b02924

- C. Pinheiro, J. P. Baeta, A. M. Pereira, H. Domingues, C. P. Ricardo, J. Food Comp. Anal. 2010, 23(4), 319–325. DOI: 10.1016/j.jfca.2010.01.005

- K. I. Ereifej, G. N. Al-Karaki, M. K., Hammouri, Int. J. Food Prop. 2001, 4(2), 239–246. DOI: g/10.1081/JFP-100105190

- FAOSTAT, 2020. Food and Agriculture Organisation (FAO) of the United Nations. Crops. Avaialble form: http://www.fao.org/faostat/en/#data/QC (Accessed on 8/4/2021).

- G. Alandia, C. Pulvento, M. H. Sellami, N. Hoidal, T. Anemone, E. Nigussie, J. J. Agüero, A. Lavini, S. E. Jacobsen, in: A. Hirich, R. Choukr-Allah, R. Ragab (Eds.): Emerging Research in Alternative Crops, Environment & Policy, vol 58. Springer, Cham, 2020, 25–53. DOI: 10.1007/978-3-319-90472-6_2

- M. Markiewicz-Keszycka, R. Cama-Moncunill, M. P. Casado-Gavalda, C. Sullivan, P. J. Cullen, Curr. Opin. Food Sci. 2019, 28, 96–103. DOI: 10.1016/j.cofs.2019.10.002

- S. Kumar, G. Pandey, Heliyon 2020, 6(3), p.e03682. DOI: 10.1016/j.heliyon.2020.e03682

- B. M. Freire, R. M. Pereira, C. N. Lange, B. L. Batista, in: Sustainable Solutions for Elemental Deficiency and Excess in Crop Plants. Springer, Singapore. 2020, pp. 135–182. DOI: 10.1007/978-981-15-8636-1_7

- N. Petry, E. Boy, J. P. Wirth, R. F. Hurrell, Nutrients, 2015, 7(2), 1144–1173. DOI: 10.3390/nu7021144

- M. A. Zoroddu, J. Aaseth, G. Crisponi, S. Medici, M. Peana, V. M. Nurchi, J. Inorg. Biochem. 2019, 195, 120–129. DOI: 10.1016/j.jinorgbio.2019.03.013

- H. Zhao, J. Tang, Q. Yang, J. Food Comp. Anal. 2021, p.103900. DOI: 10.1016/j.jfca.2021.103900

- M. Ghidotti, S. Papoci, C. Dumitrascu, T. Zdiniakova, Y. Fiamegos, M. B. de la Calle Gutiñas, Talanta Open, 2021, p.100040. DOI: 10.1016/j.talo.2021.100040

- M. Nečemer, P. Kump, J. Ščančar, R. Jaćimović, J. Simčič, P. Pelicon, M. Budnar, Z. Jeran, P. Pongrac, Spectrochim. Acta B 2008, 63, 1240–1247. DOI: 10.1016/j.sab.2008.07.006

- M. Nečemer, P. Kump, K. Vogel-Mikuš, in: Use of X-ray fluorescence-based analytical techniques in phytoremediation, Nova Science Publishers, Inc., New York, USA, 2011, 331–358

- A. P. de Oliveira, B. D. S. O. Mateó, A. M. Fioroto, P. V. de Oliveira, J. Naozuka, J. Food Comp. Anal. 2018, 67, 135–140. DOI: 10.1016/j.jfca.2018.01.012

- G. E. Guild, N. G. Paltridge, M. S. Andersson, J.C. Stangoulis, Plant Soil 2017, 419(1), 457–466. DOI: 10.1007/s11104-017-3352-4

- G. R. Gomes, G. H. Freiria, L. Vinicius Constantino, S. M. Alves, D. M. Zeffa, L. S. A. Takahashi, Semina: Ciênc. Agrár. 2020, 41(5), 1469–1482. DOI: 10.5433/1679-0359.2020v41n5p1469

- T. Celmeli, H. Sari, H. Canci, D. Sari, A. Adak, T. Eker, C. Toker, Agronomy 2018, 8(9), p.166. DOI: 10.3390/agronomy8090166

- S. Lukin, Vestnik Russian Agric. Sci. 2018, 6, 76–79. DOI: 10.30850/vrsn/2018/6/76-79

- C. I. Lizarazo, M. Yli-Halla, F. L. Stoddard, Nutr. Cycl. Agroecosystems 2015, 103(3), 311–327. DOI: 10.1007/s10705-015-9743-0

- H. Khazaei, A. Vandenberg, Agronomy 2020, 10(4), p.511. DOI: 10.3390/agronomy10040511

- Z. Türk, J. Plant Nutr. 2020, 43(4), 563–577. DOI: 10.1080/01904167.2019.1685099

- G. Ciurescu, I. Toncea, M. Ropotă, M. Hăbeanu, Rom. Agric. Res. 2018, 35, 101–108. https://www.incda-fundulea.ro/rar/nr35/rar35.14.pdf

- L. X. Lopez-Martinez, in: H. N. Murthy, K. Y. Paek (Eds.) Bioactive Compounds in Underutilized Vegetables and Legumes. Reference Series in Phytochemistry. Springer, Cham, 2020, 1–17. DOI: 10.1007/978-3-030-44578-2_31-1

- M. T. Mosisa, Afr. J. Food Sci. 2017, 11(3), 74–81. DOI: 10.5897/AJFS2016.1475

- E. M. Flores, P. A. Mello, S. R. Krzyzaniak, V. H. Cauduro, R. S. Picoloto, Rapid Commun. Mass Spectrom. 2020, 34, p.e8727. DOI: 10.1002/rcm.8727

- M. W. Donner, T. Siddique, Can. J. Chem. 2018, 96(8), 795–802. DOI: 10.1139/cjc-2017-0637

- G. G. A. de Carvalho, M. B. B. Guerra, A. Adame, C. S. Nomura, P. V. Oliveira, H. W. P. de Carvalho, D. Santos, L. C. Nunes, F. J. Krug, J. Anal. Atom. Spectrom. 2018, 33(6), 919–944. DOI: 10.1039/C7JA00293A

- E. de Almeida, N. M. Duran, M. H. Gomes, S. M. Savassa, T. N. da Cruz, R. A. Migliavacca, H. W. Pereira de Carvalho, X‐Ray Spectrometry 2019, 48(2), 151–161. DOI: 10.1002/xrs.3001

Corresponding author: e-mail: lovro.sinkovic@kis.si, phone: +386 (0)1280 52 78