Development of green differential pulse voltammetric strategy for the determination of alfuzosin hydrochloride at pencil graphite and modified carbon paste electrodes

Youstina Mekhail Metias,1 Emad Mohamed Hussien,2* Mervat Mohamed Hosny,1 and Magda Mohamed Ayad1

1 Department of Analytical Chemistry, Faculty of Pharmacy, Zagazig University 44519, Zagazig, Egypt.

2 Department of Pharmaceutical Chemistry, National Organization for Drug Control and   Research (NODCAR), Giza, Egypt.

* Corresponding author: E-mail address: emadhussien@yahoo.com

Tel.: +2 02 3749 6077

Abstract

A sensitive and inexpensive differential pulse voltammetry technique (DPV) was applied to investigate electrochemical behavior of alfuzosin hydrochloride at two different working electrodes; silica gel modified carbon paste and pencil graphite electrodes (PGE). The voltammetric conditions were optimized using cyclic voltammetry showing an irreversible anodic peak in Britton-Robinson buffered medium (pH 6) at around 0.86-0.90 V. The DPV responses were linearly correlated with alfuzosin concentrations in the ranges of 0.6- 20 and 0.3-20 µM exhibiting higher electrocatalytic activity at PGE. The proposed method was a successful attempt for drug determination in tablets formulation and spiked human urine with greenness evaluation studies.

Keywords:  alfuzosin hydrochloride; carbon paste electrode; cyclic voltammetry; differential pulse voltammetry; pencil graphite electrode.

1. Introduction

Alfuzosin hydrochloride, ALF as a selective alpha1-adrenoceptor antagonist, is defined as (2RS) -N- [3-[(4-Amino-6,7 dimethoxyquinazolin-2-yl)methylamino]propyl] tetrahydrofuran-2-carboxamide hydrochloride.1 Its action as vasodilator may be less frequent, while it acts more selective on the smooth muscle tone within the prostate and the bladder neck causing relaxation of these muscles. Consequently, this effect results in symptomatic relief of the benign prostatic hyperplasia ,a common progressive disease encountered in aging men, within weeks.2 So, after its oral administration as hydrochloride, it alleviates symptoms of urinary obstruction through reducing outflow resistance and enhancing bladder emptying.

Concerning ALF, several methods were also published for the determination of ALF  alone or in combinations including flow injection analysis,3 spectrophotometry4, 5 and spectrofluorimetry6, 7 with stability indicating studies.8 Chromatographic HPLC methods with either UV9, 10 or MS/MS11 or fluorescence12, 13 detections as well as  UPLC,14 chiral HPLC,15 HPTLC9 were also applied.

Electrochemical methods were applied for ALF determination in biological fluids including potentiometric,16 voltammetric17-21 as well as conductometric measurements in pharmaceuticals.22

To our knowledge from the literature review as depicted in Figure 1, few studies concerning electrochemical behavior of alfuzosin had been mentioned. While most researches directed towards analysis of ALF using chromatographic and spectroscopic methods, such methods either need instrumentations of high cost, high solvent use, hazardous organic solvents or complicated steps as sample pretreatment or prolonged analysis time. Therefore, the development of analytical methods having the inherent features of being simple, rapid, inexpensive, necessitating fewer reagents, easily operated but still sensitive and selective is widely demanded for the drug analysis. Amongst the available approaches, electrochemical techniques as voltammetric methods are considered satisfactory alternative23 and an appealing choice for pharmaceutical analysis24. Although ALF is an electroactive compound, few voltammetric studies were reported for its analysis employing various working electrodes as Hg electrode,17 bare,18 modified  glassy carbon electrode  with hybrid of ionic liquid- ZrO2 in graphene oxide19  and multiwall carbon nanotubes-nickel oxide nanoparticles21 as well as  the multi-walled carbon nanotube ionic liquid nanocomposite electrode.20

Figure 1: Classification of the reported analytical techniques for ALF determination

However, the reported voltammetric methods showed good sensitivity and satisfactory analytical performance, some of the applied electrodes showed some drawbacks such as mercuric toxicity, use of expensive modifiers such as multiwalled carbon nanotubes and nanoparticles as well as complicated procedures required for preparation those modified electrodes with time consuming. Therefore, continuous development and chemical modification of electrochemical sensors of low-cost and eco-friendly material, without compromising analytical performance has received extensive interest for their application as a chemical and biological sensor in the voltammetric assessment of different drugs.

Carbon paste electrode (CPE), a widely used carbon electrode, has been applied in the areas of electrochemistry and electroanalysis related to its attractive characteristics including ease of preparation, affordable implementation, easy-to-made surface renewal, a low background current and a wide range of potential window.23, 25 Also, CPE can be easily modified and simply manipulated in order to obtain a stable response with a possibility of lowering the over potential and increase sensitivity and selectivity of some electroactive species.26, 27

In this work, chemically modified graphite electrodes with silica gel have been investigated for determination of ALF. Silica gel is a granular and porous form of silicon dioxide and can be easily incorporated into carbon paste as an inexpensive and suitable modifier. It possesses some attractive electrochemical properties such as high surface area, strong adsorption capacity, insolubility in most solvents, high thermal stability, readily modified surface so, it is extremely useful for electroanalytical purposes.28-30

In recent years, disposable electrodes composed of matrixes susceptible to easy surface renewal as pencil graphite electrodes (PGE) are highly recommended for quantification assays.31-33 PGE stand as an excellent versatile tool in electroanalysis which have favorable advantages with respect to traditional electrodes of being simple, cheap, commercially available, easily modified and possessing low technology with good mechanical rigidity. PGE also showed faster electrode preparation between analyses than polishing procedures performed in common solid electrodes as glassy carbon electrodes.34 Furthermore, PGE are easily used and more convenient because there is no time consuming for electrode preparation such as tedious hand mixing step and hazardous steps observed in the preparation of conventional electrodes as graphene oxide.

Differential pulse voltammetry (DPV) as a simple pulse voltammetric technique was extremely applied for determination of various pharmaceuticals.24 Herein, DPV method was useful for studying of electrochemical oxidation of ALF at two different electrochemical sensors; silica gel-modified carbon paste electrode and pencil graphite electrode and comparing performance of both electrodes.

Thus, our study aimed to develop an easy, simple and rapid DPV method for highly sensitive quantitation of ALF in tablets and biological samples. It is the first time in the literature to apply disposable pencil graphite electrode for voltammetric assessment of ALF and to assay ALF directly in urine samples using two simple electrodes of low cost. Moreover, the ecological impact of the proposed method was also evaluated using three metrics, namely; the National Environmental Method Index (NEMI), the analytical Eco-Scale score method and the Green Analytical Procedure Index (GAPI).

2. Experimental

2.1. Instrumentation

The electroanalytical study was performed using SP-150 potentiostat (Bio-Logic Science Instrument, France) connected to a Lenovo computer provided with EC-Lab for windows v11.02 software. The cell potentials were measured with respect to Ag/AgCl (3.0 M KCl) as a reference electrode (BAS, USA) in a glass cell comprising a working electrode and a platinum wire (BAS, USA) as an auxiliary electrode.

Digital analyzer pH meter (Jenway 3510, USA), ultrasonic bath sonicator (UCI-750, RAYPA, Spain) were used.

2.2. Working electrodes

Voltammetric measurements were performed on two different electrodes;

2.2.1. Modified Carbon- Paste Electrode (CPE)

Silica gel modified carbon paste electrode (Si-gel/CPE) electrode was made by firm hand mixing of 0.2 g silica gel and 0.8 g of graphite powder with 0.6 mL paraffin oil in glass mortar by a ceramic pestle. Likewise, plain or bare (unmodified) carbon paste electrode (CPE) was prepared by blending 1.0 g of graphite powder with 0.6 mL paraffin oil. A portion of the prepared carbon paste was packed into the electrode body and followed by polishing its external surface in a figure eight motion on soft paper till obtaining a shiny appearance prior using. After every measurement, a small amount of the paste was scraped out from the electrode tip and replaced by a new carbon paste to generate a fresh electrode surface.

2.2.2. Pencil graphite electrode (PGE)

2B XQ pencil leads with 60 mm length and a diameter of 0.9 mm from the local bookstore were employed. An insulating tape was used for wrapping pencil graphite electrode gently to form about 25 mm at one end for the holder connection and only 10 mm exposed at the other end as a sensitive part for the voltammetric assay. Electrical contact of the electrode with the device was achieved by a home-designed brass holder. The exposed surface of electrode was gently polished before each recording using a cloth felt pad.

2.3. Materials and reagents

Pure sample of alfuzosin hydrochloride was generously provided by Eva Pharma, Egypt (lot no. 1422R118) with purity of 100.80% according to the comparison method.1

Chemicals of highest purity were used in this study. Graphite powder, paraffin oil, Sephadex G-50, C18 silica gel and chitosan were supplied from Sigma–Aldrich. Glacial Acetic, boric, phosphoric and hydrochloric acids, methanol and sodium hydroxide were obtained from El Nasr Pharmaceutical Chemicals CO. (Cairo, Egypt). Bi-distilled water obtained by a Milli-Q water purification system was used throughout the experiment.

Britton-Robinson buffer (B-R buffer) was prepared from a mixture of 0.04 M of each acid; boric, phosphoric, acetic acids and the desired pH was adjusted using NaOH.

2.4. Pharmaceutical formulation

Prostetrol® modified release tablets (10 mg ALF/ tablet), produced by Eva Pharma with batch no. (10)190238, were purchased from the Egyptian market to be analyzed by the proposed method.

2.5. Stock and working standard solutions

Stock standard solution of ALF (1.0 x 10−2 M) was prepared using methanol for dissolving 42.59 mg of ALF well into a 10 ml volumetric flask. Further dilution was carried out by transferring 0.25 ml of standard solution into 25 ml volumetric flask and completed up to the mark with bi-distilled water to obtain standard solution with concentration of 1.0 x 10-4M. The working solutions of ALF were made by further dilution with B-R buffer solution to cover the concentration range of 6 x 10−7 to 2 x 10−5 M and 3 x 10−7 to 2 x 10−5 M at Si-gel/CPE and PGE, respectively.

2.6. General procedure

2.6.1. Voltammetric procedure

Surface of the working electrode was conditioned by performing successive anodic CV scans within the potential of 0 up to 1.6 V in an electrochemical cell containing B-R buffer solution (pH 6.0). After achieving stable background and response, appropriate volume of ALF solution was added to the cell. At an open circuit potential, the solution was stirred for 1 min. and left for 30 s as a rest period, then the voltagramms were recorded at ambient temperature between at a scan rate of 200 mVs−1.

2.6.2. Construction of calibration curves

Quantitative determination of Alf were performed using DPV method within a potential range from 0 to 1.3 V and with pulses height 50 mV, pulses width 50 ms, step height 50 mV and step time 100 ms as the optimum DPV parameters. Two calibration curves for ALF were constructed in bulk powder covering concentration ranges from 0.6- 20 and 0.3-20 µM at Si-gel/CPE and PGE respectively. Different aliquots of ALF from the stock standard solution (1.0 x 10-4M) were transferred by a micropipette to an electrochemical cell containing 10 mL buffer (pH 6.0). Stirring of the cell content was done for 1 min. followed by measuring peak current. The peak of oxidation current (µA) developed at each working electrode was plotted against its corresponding drug concentration of ALF (µM), and then the linear regression equations were computed.

2.6.3. Assay of pharmaceutical preparations

Ten Prostetrol® tablets were finely pulverized, an equivalent amount to 1.0 x 10−2 M of ALF was accurately weighed and transferred into 25 mL volumetric flask. The flask content was dissolved in 20 mL methanol using sonication for 30 min. in an ultrasonic bath. The final volume was adjusted with the same solvent with good mixing then allowed to settle for about 15 min before filtration of supernatant.35 Further dilution was also made with bi-distilled water to obtain 1.0x10-4M solution of ALF. Different aliquots from this solution were subjected to the voltammetric measurements as described under '' construction of calibration curves'' and the recovery of each concentration was calculated using the corresponding regression equations.

2.6.4. ALF assay in spiked human urine

Fresh urine sample was collected from a healthy donor and stored at –20°C. The urine sample was allowed to be partially thawed at room temperature prior to the analysis and the clear supernatant was collected. In order to reduce the matrix effect, the urine supernatant underwent fifty and twenty folds dilution with B-R buffer of pH 6 for the voltammetric measurements at Si-gel/CPE and PGE respectively. The test solution was transferred to the voltammetric cell and treated as a blank solution. After recording a blank voltammogram, the test solution was spiked with appropriate aliquots of 1.0x10-4M ALF solution and directly analyzed using procedures described under '' construction of calibration curves'' for the recovery studies.

3. Results and discussion

The electrochemical behavior of ALF was investigated at two different electrodes; CPE and PGE by recording the CV from 0 to 1.6 V in B-R buffer as a supporting electrolyte. Different chemical and electrochemical parameters were thoroughly studied to optimize voltammetric behavior of ALF using CV. Upon applying CV technique, one anodic peak current appeared at around 0.90 V without cathodic peak on the reverse scan. It indicated that the electrooxidation of ALF is irreversible process and the prominent anodic peak was related to electroactive nitrogen atom in the amine group of the pyrimidine moiety, followed by deprotonation and dimerization. This reasonable mechanism was proposed for the previous electroxidation studies of ALF.20, 21

3.1. Optimization of the experimental Conditions using cyclic voltammetry

3.1.1. Effect of pH

The pH of the supporting electrolyte is considered as one of most common and strongest variables affecting voltammetric behavior of the drug. Hence, the effect of changing pH range of B-R buffer from 3to 9 was studied on the electrooxidation peak of 10 μM ALF. Upon increasing the pH, the anodic peak potential was shifted toward less positive direction and followed this linear equation (Figure 2B);

EPa = –0.0561pH + 1.3287 (R2 = 0.990)                                                                                  (1)

that proved proton presence in the oxidation process. Moreover, the slope of 0.0561V/pH indicated that equal number of electrons and protons participated in the process of drug oxidation as this value showed a good agreement with the theoretical value of 0.059 V /pH respecting the Nernst equation. Meanwhile, the peak current enhanced in less acidic medium till pH 6, after which badly-defined oxidation peaks appeared with gradual decrease in the peak height. Thus, B-R buffer at pH 6.0 was chosen as the electrolyte medium for all subsequent measurements of ALF, Figure 2.

Figure 2: (A) Cyclic voltammograms recorded at different pH values (3-9) of B-R buffer containing 10μM of ALF with 200 mV/s scan rate, (B) dependence of peak potential and peak current as a function of pH values

3.1.2. Effect of different additives to the supporting electrolyte

Oxidation behavior of ALF was studied in the presence of different types of surfactant including SDS (anionic), CTAB (cationic), and Brij-35 (nonionic surfactant), as a well-known additive in the electroanalytical chemistry. Figure 1S showed no distinct effect on the electrochemical behavior of ALF when the surfactant was added to the analyte in the B-R buffer solution (pH6) and the aqueous electrolytic solution of B-R buffer gave the best response.

3.1.3. Effect of stirring time

To study the stirring time effect at 400 rpm on the peak current, the DPV was recorded after stirring the solution at different time intervals from 0 to 3 min. at an open circuit potential followed by 30 s quiescent time. As shown in Figure S2, it was found that stirring for 1 min., before any measurements, improved the oxidation current intensity while increasing the stirring time from 1 to 4 min. showed no different response.

3.1.4. Effect of Silica gel

Different materials were added to the carbon paste electrode including silica gel, chitosan, sephadex and iron nanoparticles to test possibility of enhancing the oxidation current and hence sensitivity of the electrode. The electrode response was boosted upon modification of the carbon paste with silica gel which indicated the high affinity of the studied drug to the surface of the modified paste and led to high sensitivity to ALF, Figure S3. The silica gel content in the carbon paste was studied and the addition of silica gel (20% w/w of the total mass of the paste) resulted in a considerable improvement in the oxidation peak current, , which is most likely due to the synergistic effect of silica gel, Figure S4.

3.1.5. Effect of type of pencil graphite

Different diameters and hardness of disposable pencil leads from different commercial manufacturers (HB, rotring, faber castle and XQ) were examined using CV for 5 µM ALF. The best electrochemical response, in terms of height of peak current, was found to be 2B XQ pencil lead of 0.9 mm diameter (Figure S5).

3.1.6 Optimization of scan rate

Studying the influence of the scan rate gave valuable information about the electrochemical behavior of the studied drug at the electrode surface indicating whether the electooxidation process was under diffusion or adsorption controlled mechanism. The variation in the peak current (ip) and peak potential (Ep) of 10 µM ALF in B-R buffer solution (pH 6) was recorded upon raising the scan rate from 25 up to 300 mV/s using cyclic voltammetric mode.  It was observed that upon increasing the scan rate, the anodic peak current grew gradually and the oxidation peak potential shifted slightly towards more positive direction indicating irreversible oxidation process. The scan rate of 200 mV/s was selected for successive voltammetric measurements giving relatively high sensitivity with well-shaped and relatively narrow peak width, Figure 3 A.

A direct relationship was found between the oxidation peak current of ALF and the square root of scan rate (υ1/2) over the range of 25– 200 mVs−1, based on I(μA) = 2.7885 υ1/2 (mV/s)+ 2.0843(R2 = 0.9816)  exhibiting that the electrochemical oxidation of ALF was controlled by a diffusion process, Figure 3 B.

Moreover, plotting logarithm of the peak current versus logarithm of scan rate exhibited a straight line following the equation (Figure 3 C);

log Ipa (μA) = 0.4932 log υ (mV/s) + 0.4922, R2 = 0.98                                                        (2)

The obtained slope value of 0.4932 was close to the theoretical value of 0.5 for the diffusion-controlled electrode process.36

Furthermore, the relationship between the peak potential and logarithm of the scan rate was found to be linear according to the following equation (Figure 3 D);

Epa(V)= 0.0619 logυ (V/s)+ 1.0738, R2 = 0.9909                                                                   (3)

In accordance with Laviron᾿s equation (1) given for an irreversible electrochemical process, the number of electrons (n) transferred at the surface of the electrode due to ALF oxidation was calculated as follows;37

Ep=E°+2.303RTαnF(logRTk°αnF)+2.303RTαnFlogυ                                                                           (4)

Here, the following elements in the equation possess their conventional meanings; the electron transfer coefficient (α), the number of electrons transferred (n), temperature (T =298K), gas constant (R = 8.314 J K mol−1), Faraday constant (F = 96 485 C mol−1) and the voltammetric scan rate (υ). E° is the formal redox potential obtained from the intercept of potential against scan rate curve by extrapolating to the vertical axis at υ = 0 and k° is the standard heterogeneous rate constant of the surface reaction. From the slope of the linear relationship between Epa versus log υ, the value of αn was obtained and found to be 0.0619. The supposed value of α equals 0.5 for the irreversible electrochemical process,38 thus the n value was calculated to be 1.911 (approximately 2.0). This value referred to the participation of two electrons and protons in the electrooxidation reaction of ALF which coincided with previously published results.20, 21

Figure 3: A; Cyclic voltammograms of 10 μM of ALF in B-R buffer of pH 6 at different scan rates with the inset of linear relationship between peak current and peak potential. B; dependence of the peak current on square root of the scan rate, C; dependence of the peak current logarithm on the scan rate logarithm and D; dependence of the peak potential on the scan rate.

3.2. Method validation

The developed DPV method at two different electrodes was validated according ICH guidelines39 with regard to the following parameters;

3.2.1. Linearity and range

Under the described conditions, the linearity of ALF was investigated over the concentration ranges from 6 x 10−7 to 2 x 10−5 M at Si-gel/CPE and from 3 x 10−7 to 2 x 10−5 M at PGE by DPV, Figure 4. Statistical analysis of the differential pulse voltammograms was performed by plotting the peak current height (µA) versus concentrations (µM). The calibration graphs showed linear behavior with high value of the correlation coefficient and represented by the following equations;

Si-gel/CPE: I=0.2501 C + 2.7407 (R2 = 0.9995)                                                                     (5)

PGE: I= 0.6057 C + 3.3274 (R2 = 0.9996)                                                                              (6)

The obtained statistical data proved good linearity of the method and included in Table 1.

Figure 4: DPV voltammograms of A; 0.6 – 20 µM of ALF at Si-gel/CPE, B; 0.3 - 20 µM of ALF at PGE in B-R buffer solution of pH 6 and C; their corresponding calibration graphs for ALF assay.

| Table 1. Regression and analytical performance data for voltammetric estimation of ALF at two different electrodes by DPV method. |  |  |
| --- | --- | --- |
| # Parameters | Si-gel/CPE | PGE |
| Linearity range (μM) | 0.6-20 | 0.3-20 |
| Intercept | 2.7407 | 3.3274 |
| Slope | 0.2501 | 0.6057 |
| Correlation Coefficient(r2) | 0.9995 | 0.9996 |
| LOD (µM) | 0.17 | 0.099 |
| LOQ (µM) | 0.53 | 0.299 |
| Accuracy ) |  |  |
| (mean recovery% ± Er%) | 100.85+0.85 | 98.97-1.03 |
|  | 97.57-2.43 | 99.74-0.26 |
|  | 98.88-1.01 | 100.94+0.94 |
| Precision (RSD%) |  |  |
| Repeatability[a] | 1.756 | 1.399 |
| Intermediate precision[b] | 1.794 | 1.788 |
| [a] The intra-day and [b] the inter-day RSD (n=9) of determining 1, 10 and 20 μM of ALF at Si-gel/CPE or 1, 10 and 15 μM of ALF at PGE within the same day and in three successive days. |  |  |

3.2.2. Sensitivity

Limits of detection (LOD) and quantification (LOQ) were calculated according to the ICH guidelines;39 LOD = 3.3 σ/s and LOD = 10 σ/s (where, σ is the standard deviation of four replicates of the blank obtained in the same potential oxidation of the sample and s is the slope of the calibration graph). The values of LOD and LOQ were listed in Table1 indicating better sensitivity at both electrodes.

3.2.3. Accuracy and precision

The accuracy of the proposed methods was examined using nine samples of three different concentrations of the pure drugs selected to cover low, medium and high ranges of the calibration graph. The mean percentage recoveries of these concentrations of ALF were calculated, as shown in Table 1.

The precision was also evaluated through triplicate determinations of three different concentrations of each pure drug in the same day (intra-day) and in three different days (inter-day). Percentage relative of error (Er%) and standard deviation (RSD%) were calculated and exhibited acceptable results  in Table 1.

3.2.4. Specificity

The specificity of the proposed method was confirmed through the assay of ALF in urine and in its commercial tablet. The selectivity was evaluated in the presence of some common excipients widely found in tablet dosage form such as silica, lactose, PVP, talc and magnesium stearate which were added to the drug in the same concentration of 10-5 M. Table 2 showed sufficient good recoveries with no interference while lactose exibited only slight decrease in the electrochemical response of ALF.

| Table 2: Voltammetric analysis of ALF at two different electrodes in the presence of some common excipients. |  |  |
| --- | --- | --- |
| Excipients Added[a] (1x 10-5 M) | Recovery %[b] |  |
|  | Si-gel/CPE (1x 10-5 M) | GPE (1x 10-5 M) |
| Silica | 103.40 | 101.88 |
| Lactose | 95.00 | 94.23 |
| Povidone k30 | 102.87 | 97.67 |
| Talc | 100.10 | 100.40 |
| Magnesium stearate | 101.18 | 101.71 |
| [a] Drug: excipients in the ratio of 1: 1 M. [b] Mean of three determinations. |  |  |

3.3. Statistical Analysis

The results of the proposed methods were statistically examined using Student’s t-test and variance ratio F-test at 95% confidence level. The obtained results were compared with those of the potentiometric official method for ALF.1 Table 3 showed no significant differences between the developed method and the pharmacopeial one.

| Table 3. Statistical analysis of results obtained by the proposed method for ALF at two different electrodes and the pharmacopeial method |  |  |  |
| --- | --- | --- | --- |
|  | Si-gel/CPE | PGE | Comparison method [1][a] |
| Mean ± SD | 100.10±1.584 | 99.39±1.404 | 100.80±1.420 |
| Variance | 2.508 | 1.970 | 2.016 |
| n | 6 | 7 | 3 |
| Student-t-test | 0.642 (2.365)[b] | 1.549(2.306)[b] | ------ |
| F-test | 1.244 (5.790)[b] | 1.023 (5.140)[b] | ------ |
| [a] Potentiometric official method for ALF. [b] The corresponding theoretical values for t and F tests at p = 0.05. |  |  |  |

3.4. Analytical applications

The employed electrodes showed satisfactory results for assay of ALF in Prostetrol® tablets and spiked urine samples by the differential pulse voltammetry. The recovery values of target analytes and standard deviations (SD) presented in Tables 4 proved suitability of the proposed voltammetric procedures for fast routine analysis of ALF in commercial tablets and human urine samples.

| Table 4. Application of the DPV method for determination of ALF in tablets formulation and spiked urine sample at two different electrodes |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Si-gel/CPE |  |  |  |  |  | PGE |  |  |  |  |  |
|  | Prostetrol® modified release tablets |  |  | Urine samples |  |  | Prostetrol® modified release tablets |  |  | Urine samples |  |  |
| Parameters | Taken μM | Found μM | Recovery [a]% | Taken μM | Found μM | Recovery[a]% | Taken μM | Found μM | Recovery[a]% | Taken μM | Found μM | Recovery [a]% |
|  | 5 | 4.02 | 101.92 | 3 | 3.08 | 102.57 | 0.6 | 0.59 | 98.43 | 5 | 5.10 | 102.01 |
|  | 7 | 4.49 | 99.96 | 5 | 5.12 | 102.36 | 5 | 4.89 | 97.74 | 7 | 7.19 | 102.66 |
|  | 13 | 6.05 | 101.72 | 10 | 10.10 | 100.95 | 7 | 6.97 | 99.61 | 10 | 10.02 | 100.24 |
|  | 16 | 6.82 | 101.97 | 15 | 15.15 | 101.03 | 10 | 10.04 | 100.43 | 12 | 11.80 | 98.30 |
|  | 20 | 7.84 | 102.00 |  |  |  | 12 | 11.65 | 97.10 |  |  |  |
| Mean ±SD | 101.51±0.876 |  |  | 101.73±0.854 |  |  | 98.66 ± 1.357 |  |  | 100.94 ± 1.731 |  |  |
| N | 5 |  |  | 4 |  |  | 5 |  |  | 4 |  |  |
| SE | 0.392 |  |  | 0.427 |  |  | 0.607 |  |  | 0.866 |  |  |
| V | 0.767 |  |  | 0.729 |  |  | 1.842 |  |  | 2.997 |  |  |
| [a] mean of three different experiments. |  |  |  |  |  |  |  |  |  |  |  |  |

3.5. Greenness profile of the proposed method

Assessment of analytical method procedures from green perspective view has recently attracted authors’ concern in the field of green analytical chemistry(GAC). Electroanalytical techniques mostly comply with the principles of GAC as they don't require hazardous or harmful chemicals or organic solvents and don't produce large volume of analytical waste compared to classical chromatographic methods.

To deepen this view, the greenness profile of the developed method was established using three assessment tools relating to GAC to inspect its impact on the environment. The assessment profiles by NEMI, Analytical Eco-Scale and GAPI methods introduce more easier and visible information on the analytical procedure.

The applied NEMI labeling is considered as one of the oldest tools for qualitative greenness assessment of the analytical procedures. The NEMI pictogram is symbolized by a circle divided into four fields reflecting four different criteria of the described analytical methodology: PBT (persistence, bioaccumulation potential and toxicity); hazardous chemicals; corrosiveness; and waste.  Each field is filled green when its required criterion is met by the developed method.40 Adopting these criteria, the applied method showed green-colored four quadrants due to satisfaction the four requirements as the pictograms in Table 5.

From a glance at the NEMI symbol, general information about environmental impact of the analytical procedure can be easily read with no significance to energy consumption or the quantity of chemicals. So the analytical Eco-Scale, further approach for assessing the greenness in a more quantitative way, has been also used where a numerical score is given for the applied method.  The analytical Eco-Scale score is calculated from subtraction of total penalty points from the basis of 100 points (ideal green analysis). Penalty points of the analytical procedure are assigned for the amount and type of chemical reagents, energy consumption, occupational hazard, amount of generated waste and the way for its treatment. The score value is ranked on a scale where the green analysis method is deemed excellent if the score is higher than 75; acceptable if it is higher than 50 and inadequate if it is less than 50.41 The DPV method scored 80 at Si-gel/CPE  and 82 at PGE (higher than 75) So it ranked as an excellent green method as shown in Table 5 which demonstrated the calculation of analytical Eco-Scale scores in detail for assessing its greenness profile more obvious than that obtained by NEMI.

The greenness assessment has been also achieved according to a recent tool known as GAPI which have a specific symbol with five pentagrams involving 15 zones to encompass five main categories; sample handling, general method type, sample preparation, reagents/solvents used and instrumentation. The GAPI approach evaluates 15 criteria covering every step in the whole analytical procedure through using a color code; green, yellow and red signifying low, medium and high ecological impacts respectively.42 The GAPI pentagram of the proposed method showed (6) green, (7) yellow, and (2) red shaded zones reflecting its lower impact on the environment, Table 5.

All in all, this simple study is shed light on the merits of the applied voltammetric method of being greener and safer practice for quantification of ALF to be used in routine work and quality control purposes in pharmaceutical industries.

| Table 5. Greenness assessment of the proposed method by the Analytical Eco-Scale, NEMI and GAPI approaches. |  |  |
| --- | --- | --- |
| Analytical Eco–Scale score parameters | Penalty points (PPs) | NEMI pictograms |
| I–Reagents/word sign/no of pictograms |  |  |
| Bi-distilled water/ - / 0 | 0 |  |
| Methanol/ danger/ 2 | 4 |  |
| 1M NaOH/ danger/ 1 | 2 |  |
| Silica gel / -/ 0 | 0 |  |
| Paraffin oil/ danger/ 1 (for Si-gel/CPE) | 2 |  |
| Graphite/ -/ 0 | 0 |  |
| 0.04 M acetic acid / danger / 1 | 2 |  |
| 0.04 M orthophosphoric acid/ danger/ 1 | 2 |  |
| 0.04 M boric acid/ danger/ 1 | 2 |  |
|  | ∑=14 |  |
| II–Instruments |  | GAPI pictograms* |
| a-Energy |  | * Red zones depict high ecological impact; yellow zones represent lower impact; and green zones represent more safe effect to environment. |
| Potentiostat | 0 |  |
| pH meter | 0 |  |
| Vortex mixer | 0 |  |
| Sonicator | 0 |  |
| b–Occupational hazards | 3 |  |
| c–Waste | 3 |  |
|  | ∑= 6 |  |
| Total PPs | 20 |  |
| Analytical eco–scale score = 100- total PPs | 80 |  |
|  | Excellent green analysis |  |

4. Conclusion

Herein, differential pulse voltammetric method was applied for sensitive, rapid and simple quantitative analysis of ALF using two simpler and inexpensive electrodes relative to the previous published methods. Carbon paste electrode modified with silica gel and pencil graphite electrode were employed to study the electrochemical behavior of ALF in B-R buffer and showed well- defined irreversible anodic peak with optimal current density at pH 6. The analytical procedures was fully validated and applied for determination of ALF in commercial tablets and human urine samples with satisfactory recoveries. It is the first time to employ voltammetric method for assay ALF in urine sample with no need of sample extraction or separation or use hazardous materials. The sensitivity of ALF towards PGE and CPE with respect to using the same technique, ALF showed remarkable electrochemical activity towards PGE with no surface modification and it is also the first time to use simple disposable PGE for fast green analysis of ALF.

5. References

1. BP, Her Majesty's Stationery Office, London, 2021, pp. 96-98.

2. S. C. Sweetman, Book Martindale : the complete drug reference The Pharmaceutical Press, London; Chicago, 2014.

3. N. A. Mohamed, S. Ahmed and S. A. El Zohny, J. Fluoresc. 2013, 23, 1301-1311. https://doi.org/10.1007/s10895-013-1264-0.

4. S. A. Al-Tamimi, F. A. Aly and A. M. Almutairi, J. Anal. Chem. 2013, 68, 313-320. https://doi.org/10.1134/S1061934813040102.

5. M. Krishna, Vamsi  and D. G. Sankar, J Chem 2007, 4, 397-407. https://doi.org/10.1155/2007/912762.

6. M. A. Omar, A.-M. I. Mohamed, S. M. Derayea, M. A. Hammad and A. A. Mohamed, Spectrochim. Acta, Part A 2018, 195, 215-222. https://doi.org/10.1016/j.saa.2018.01.077.

7. A. Karasakal and S. T. Ulu, J. Anal. Chem. 2015, 70, 708-711. https://doi.org/10.1134/S1061934815060040.

8. A. S. Fayed, M. A. Shehata, N. Y. Hassan and S. A. Weshahy, Pharmazie 2007, 62, 830-835. https://doi.org/10.1691/ph.2007.11.7082.

9. A. S. Fayed, M. A. A. Shehata, N. Y. Hassan and S. A. El‐Weshahy, J. Sep. Sci. 2006, 29, 2716-2724. https://doi.org/10.1002/jssc.200600220.

10. P. D. Satpute, S. B. Jadhav, M. P. Rathod and P. S. Naykodi, Pharm.l Reson. 2018, 1, 21-25.

11. J. L. Wiesner, F. C. W. Sutherland, G. H. van Essen, H. K. L. Hundt, K. J. Swart and A. F. Hundt, J. Chromatogr. B 2003, 788, 361-368. https://doi.org/10.1016/S1570-0232(03)00064-3.

12. A. K. Shakya, T. A. Arafat, A. Abuawaad, H. Al-Hroub and M. Melhim, Jordan J. Pharm. Sci. 2010, 3, 25-36.

13. YAOHong-wei, Y. Jin, J. Li, Y.-f. Zhang, X.-n. Ding and S.-y. Xu, Chin. J. Pharm. Anal. 2002, 22, 127-128.

14. M. N. Brinda, V. K. Reddy and E. S. Goud, Int. J. Pharm. Biol. Sci. 2014, 4, 54-60.

15. A. Rouchouse, M. Manoha, A. Durand and J. P. Thenot, J. Chromatogr. A 1990, 506, 601-610. https://doi.org/10.1016/S0021-9673(01)91608-5.

16. V. K. Gupta, A. K. Singh and B. Gupta, Comb. Chem. High Throughput Screening 2007, 10, 560-570. https://doi.org/10.2174/138620707782152416.

17. C. E. Xin and Z. Y. Huai, Chin. J. Pharm. 2001, 32, 362-364.

18. B. Uslu, Electroanalysis 2002, 14, 866-870. https://doi.org/10.1002/1521-4109(200206)14:12<866::aid-elan866>3.0.co;2-y.

19. H. Rashedi, P. Norouzi and M. R. Ganjali, Int. J. Electrochem. Sci. 2013, 8, 2479-2490.

20. M. R. Baezzat, F. Banavand and F. Fasihi, J. Mol. Liq. 2017, 233, 391-397. https://doi.org/10.1016/j.molliq.2017.02.119.

21. Z. Pourghobadi and R. Pourghobadi, J. Electrochem. Soc. 2019, 166, B76-B83. https://doi.org/10.1149/2.0651902jes.

22. S. Ashour and M. Khateeb, Can. Chem. Trans. 2013, 1, 292-304.

23. J. Wang, Book Analytical Electrochemistry, Wiley-VCH, New York, 2006.

24. V. K. Gupta, R. Jain, K. Radhapyari, N. Jadon and S. Agarwal, Anal. Biochem. 2011, 408, 179-196. https://doi.org/10.1016/j.ab.2010.09.027.

25. I. Švancara, K. Vytřas, K. Kalcher, A. Walcarius and J. Wang, Electroanalysis 2009, 21, 7-28. https://doi.org/10.1002/elan.200804340.

26. A. A. Al-rashdi, O. A. Farghaly and A. H. Naggar, J. Chem. Pharm. Res. 2018, 10, 21-43. https://doi.org/10.13140/RG.2.2.15392.23047.

27. E. M. Hussien, H. Saleh, M. El Henawee, A. Abou El Khair and N. Ahmed, Acta Chim. Slov. 2020, 67, 757-763. https://doi.org/10.17344/acsi.2019.5686.

28. Z. Nasri and E. Shams, Electrochim. Acta 2009, 54, 7416-7421. https://doi.org/10.1016/j.electacta.2009.07.089.

29. M. Jaafariasl, E. Shams and M. K. Amini, Electrochim. Acta 2011, 56, 4390-4395. https://doi.org/10.1016/j.electacta.2010.12.052.

30. H. E. Zaazaa, N. N. Salama, S. M. Azab, S. A. Atty, N. M. El-Kosy and M. Y. Salem, RSC Adv. 2015, 5, 48842-48850. https://doi.org/10.1039/C5RA06292F

31. H. T. Purushothama, Y. A. Nayaka, M. M. Vinay, P. Manjunatha, R. O. Yathisha and K. V. Basavarajappa, J. Sci.: Adv. Mater. Devices 2018, 3, 161-166. https://doi.org/10.1016/j.jsamd.2018.03.007.

32. I. Y. L. d. Macêdo, M. F. Alecrim, J. R. Oliveira Neto, I. M. S. Torres, D. V. Thomaz and E. d. S. Gil, Braz. J. Pharm. Sci. 2020, 56, e17344. https://doi.org/10.1590/s2175-97902019000317344.

33. P. T. Pinar, Acta Chim. Slov. 2020, 67, 212-220. https://doi.org/10.17344/acsi.2019.5367.

34. I. G. David, D.-E. Popa and M. Buleandra, J. Anal. Methods Chem. 2017, 2017, 1905968. https://doi.org/10.1155/2017/1905968.

35. USP, The United States Pharmacopeial Convention, Rockville, 2013, pp. 2366-2369.

36. D. K. Gosser, Synthesis and Reactivity in Inorganic and Metal-Organic Chemistry 1994, 24, 1237-1238. https://doi.org/10.1080/00945719408001398.

37. E. Laviron and L. Roullier, J. Electroanal. Chem. Interfacial Electrochem. 1980, 115, 65-74. https://doi.org/10.1016/S0022-0728(80)80496-7.

38. B. Hatamluyi, F. Lorestani and Z. Es’haghi, Biosens. Bioelectron. 2018, 120, 22-29. https://doi.org/10.1016/j.bios.2018.08.008.

39. ICH Harmonised Tripartite Guideline: International conference on harmonization, Geneva, Switzerland, incorporated in November 2005.

40. L. H. Keith, L. U. Gron and J. L. Young, Chemical Reviews 2007, 107, 2695-2708. https://doi.org/10.1021/cr068359e.

41. A. Gałuszka, Z. M. Migaszewski, P. Konieczka and J. Namieśnik, TrAC Trends Anal. Chem. 2012, 37, 61-72. https://doi.org/10.1016/j.trac.2012.03.013.

42. J. Płotka-Wasylka, Talanta 2018, 181, 204-209. https://doi.org/10.1016/j.talanta.2018.01.013.