Supplementary Materials

Journal: Acta Chimica Slovenica

Article Title:  Spectrophotometric Measurement of Lithium in Human Saliva Using the                                                                Chromogenic Reagent Thorin

Author: Stephen R. Levitt*

School of Medicine, Case Western Reserve University, Cleveland, Ohio 44106, USA

*Current Affiliation: Pain Resource Center, Inc., Hillsborough, NC, 27278 USA

email: stevelevitt@earthlink.net

Contents

Table S1: Infrared absorbance measurements to determine absorbance maximum for Li/Thorin complex in human saliva, plotted in Figure 1of manuscript (page 2).

Table S2: Infrared absorbance measurements to determine time of reaction completion for lithium/Thorin complex in human saliva, plotted in Figure 2 of manuscript (page 3).

Table S3: Colorimetric Monitoring of Saliva Lithium in a Hospitalized Patient during Initial Treatment Titration with Lithium Carbonate, plotted in Figure 4 of the manuscript (page 4).

Table S4: Safety and Precautions for Use of the chemical Thorin (C16H11AsN2O10S2) (page 5-6).

1

Table S1

Infrared Absorbance measurements to determine absorbance maximum for Li/Thorin complex in human saliva and plotted in Figure 1 of manuscript

Absorbance A*	        Absorbance B*	         Absorbance C

Wave Length	   0 mEq/L Lithium	   2.48 mEq/L Lithium	      2.48 mEq/L Lithium

(nm)		   vs water reference	     vs water Reference	    vs 0 mEq/L Reference Blank**

410			1.591			1.472			     ---

420			1.816			1.747			     ---

430			1.987			1.996			     ---

440			2.061			2.158			   0.111

450			2.022			2.224			   0.210

460			1.876			2.158			   0.285

470			1.683			2.001			   0.323

475			1.572			1.898			   0.334

480			1.455			1.788			   0.337

485			1.341			1.672			   0.333

490			1.234			1.546			   0.315

500			1.080			1.322			   0.242

510			0.976			1.107			   0.133

520			0.888			0.907			     ---

530			0.791			0.749			     ---

540			0.718			0.641			     ---

Absorbance was measured from 420-540 nm for test samples A, B and C with results plotted as absorption spectra A, B and C in Figure 1 in main manuscript.

*Reagent medium for Absorbance of saliva test samples A (0 mEq/L lithium), B (2.48 mEq/L lithium), and C (2.48 mEq/L lithium): 0.1 ml Saliva + 0.1 ml reagent grade 20% KOH + 3.5 ml reagent grade acetone + 1.2 ml deionized water + 0.3 ml 02% Thorin.

** Reagent medium for Reference Blank used for Absorbance C test sample: same as above except replacing 0.3 ml 0.2% Thorin with 0.3 ml deionized water.

See Methods, Figure 1, and Results and Discussion for details regarding saliva collection,  reagent media, and spectroscopy details.

2

Table S2

Infrared absorbance measurements to determine time of reaction completion for lithium/thorin complex in human saliva*

Time	             Absorbance of 2.42 mEq/L lithium

(minutes)		vs 0 mEq/L lithium Reference Blank

3			       0.326

5			       0.330

10			       0.337

15			       0.342

20			       0.346

25			       0.349

30			       0.352

35			       0.355

45	                               0.357

60			       0.359

*Absorbance measured at 480 nm and plotted in Figure 2 of manuscript.

Reagent medium for 2.42 mE/L lithium saliva test sample: 0.1 ml saliva containing 2.42 mEq/L lithium + 0.1 ml 20% reagent grade potassium hydroxide + 3.5 ml reagent grade acetone + 1.2 ml deionized water + 0.3 ml 0.2% Thorin.  Reagent medium for 0 mEq/L lithium saliva Reference Blank is identical to that for 2.42 mE/L lithium saliva test sample, except replacing 0.1 ml saliva containing 2.42 mEq/L with 0.1 ml saliva containing 0 mEq/L lithium.

See Methods, Figure 2, and Results and Discussion for details regarding saliva collection and spectroscopy details.

3

Table S3

Colorimetric Monitoring of Saliva Lithium in a Hospitalized Patient during Initial Treatment Titration with Lithium Carbonate,ª plotted in Figure 4 of the manuscript.

Day of 		LiCO₃		        Absorbanceᶜ	      Predicted Lie   Atomic Absorption

Treatment	Doseᵇ 		        vs Reagent Blankd        mEq/L	             mEq/L

4		300 mg 1x/day		1.475		         0.20		 0.45

## 5		300 mg 2x/day		1.470		         0.16		 0.32

## 6		300 mg 2x/day		-------		         -----		 -----

## 7		300 mg 3x/day		1.586		         1.07		 1.15

8		300 mg 3x/day		1.608		         1.24		 1.47

9		300 mg 3x/day		1.634		         1.45		 1.53

10		300 mg 3x/day		1.632		         1.43		 1.40

11		300 mg 3x/day		-------		         -----		 -----

12		600 mg 2x/day		1.617		         1.31		 1.30

13		600 mg 2x/day		--------		         -------		 -----

14		600 mg 2x/day		1.647		         1.55        	 1.61

15		600 mg 2x/day		1.640		         1.49		 1.59

16		600 mg 2x/day		1.637		         1.47		 1.59

ªMale patient, age 52, admitted with diagnosis of hypomanic episode.

ᵇTherapy initiated on Day 0 with LiCO₃ 300 mg 1x/day. LiCO₃ dose was increased on Day 5, 7, and 12.

ᶜSaliva was collected by Method C and absorbance measurements at 480 nm started on Day 4 of Li therapy.

Absorbance reagent: 0.1 ml Saliva + 0.1 ml 20% reagent grade KOH + 3.5 ml reagent grade acetone + 1.2 ml deionized water + 0.3 ml 0.2% Thorin solution.

dReagent blank: identical to Absorbance reagent except replaced 0.3 ml 0.2% Thorin with 0.3 ml deionized water.

eThe calibration regression equation y = 0.128 + 1.449 (see Figure 3 in manuscript) was used to predict Li concentrations.

4

Table S4

Safety and Precautions for Use of the Chemical Thorin (C16H11AsN2O10S2)

Hazzard

This chemical is considered hazardous to individuals and the environment by the 2012 OSHA Hazard Communication Standard (29 CFR 1910.1200).

Toxic if swallowed (Acute oral toxicity Category 3)

Toxic if inhaled (Acute Inhalation Toxicity - Dusts and Mists Category 3)

Very toxic to aquatic life

Very toxic to aquatic life with long lasting effects

Signal Word-Danger

Exposure Controls/Personal Protection

Eye/face Protection Wear appropriate protective eyeglasses or chemical safety goggles as described by

OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard

EN166.

Skin and body protection

Wear appropriate protective gloves and clothing to prevent skin exposure.

Handling and Storage

Handling Do not breathe dust. Do not get in eyes, on skin, or on clothing. Do not ingest. If swallowed

then seek immediate medical assistance. Handle product only in closed system or provide appropriate exhaust ventilation.

Storage Keep in a dry, cool and well-ventilated place. Keep container tightly closed.

Precautionary Statements

Prevention

Wash face, hands and any exposed skin thoroughly after handling

Do not eat, drink or smoke when using this product

Avoid breathing dust/fume/gas/mist/vapors/spray

Use only outdoors or in a well-ventilated area

Inhalation

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing

Call a POISON CENTER or doctor/physician

Ingestion

IF SWALLOWED: Immediately call a POISON CENTER or doctor/physician

Rinse mouth

Storage

Store locked up

Store in a well-ventilated place. Keep container tightly closed

Disposal

Dispose of contents/container to an approved waste disposal plant

First Aid

Eye Contact Immediate medical attention is required. Rinse immediately with plenty of water, also under

the eyelids, for at least 15 minutes.

Skin Contact Wash off immediately with soap and plenty of water while removing all contaminated

clothes and shoes. Immediate medical attention is required.

Inhalation Remove from exposure, lie down. Remove to fresh air. If not breathing, give artificial

respiration. Immediate medical attention is required.

Ingestion Call a physician immediately. Clean mouth with water.

5

Accidental Release

Personal Precautions Ensure adequate ventilation. Use personal protective equipment as required.

Environmental Precautions Do not flush into surface water or sanitary sewer system. Do not allow material to

contaminate ground water system. Prevent product from entering drains. Local authorities

should be advised if significant spillages cannot be contained. Should not be released into

the environment.

Methods for Containment and Clean Up

Wear self-contained breathing apparatus and protective suit. Sweep up and shovel into

suitable containers for disposal. Do not let this chemical enter the environment.

Toxicological Information

Acute Toxicity

Delayed and immediate effects as well as chronic effects from short and long-term exposure

Ecological Information

Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment. May cause long-term adverse effects in the environment. Do not allow material to contaminate ground water system.

This information is based on the present state of available knowledge and we cannot assume any liability for use, or lack of use, of this information.  We encourage users of the chemical Thorin and this information to conduct their own investigations regarding safety and use.

6